# Mech-Mind Programming Handbook for Machine Tending

A general engineering handbook for building vision-guided load/unload cells with Mech-Eye 3D cameras, Mech-Vision, Mech-Viz and the Standard Interface.

Reference implementation: solution MechMaster, project Drill_Tap (Mech-Vision 2.1.1, Mech-Eye NANO ULTRA, Mech-Mind IPC STD).

| Document | Mech-Mind Machine Tending Programming Handbook |
| --- | --- |
| Revision | 1.0 |
| Date | 2026-08-12 |
| Owner | Icr247 |
| Software baseline | Mech-Vision / Mech-Viz 2.1.x (observed build 2.1.1). Where only 2.2.1 ("latest") text exists, it is marked. Legacy 1.7.x names are given for older sites. |
| Hardware baseline | Mech-Eye NANO ULTRA (24 VDC / 3.75 A), Mech-Mind IPC STD (IPCW-i5-16G-512G) |

## Contents

| § | Section |
| --- | --- |
| 0 | How to use this handbook |
| 1 | Machine tending as a vision problem |
| 2 | System architecture |
| 3 | Hardware and physical design |
| 4 | Bring-up sequence |
| 5 | Camera setup for machined metal |
| 6 | Hand-eye calibration |
| 7 | The vision project |
| 8 | Step reference (machine-tending subset) |
| 9 | Fixture-side and cell-state logic |
| 10 | Motion planning: Path Planning step vs Mech-Viz |
| 11 | The Output step as an interface contract |
| 12 | Robot ↔ vision communication (Standard Interface) |
| 13 | Robot-side programs |
| 14 | Error handling and recovery |
| 15 | Cycle time engineering |
| 16 | Changeover and multiple part numbers |
| 17 | Deployment and production |
| 18 | Commissioning and acceptance |
| 19 | Maintenance and troubleshooting |
| A | Status codes |
| B | Command quick reference and legacy names |
| C | Robot routine cross-reference |
| D | Naming and project hygiene conventions |
| E | Cell-specific record (fill in per installation) |
| F | Items to verify on your build |
| G | Glossary |
| H | Sources |

## 0. How to use this handbook

### 0.1 What this is

This is a general programming handbook: it describes how to build any machine-tending application on the Mech-Mind stack — loading and unloading CNC lathes, mills, drill/tap machines, presses, deburring cells, washers — rather than documenting one finished project. The Drill_Tap project is used throughout as the worked reference, because it already contains the canonical pattern: capture → recognise → gate → filter → sort → reorder → trim → plan → output.

Read it in one of three ways:

| You are | Read |
| --- | --- |
| Designing a new cell | §1 → §2 → §3 → §4, then use §18 as the schedule |
| Building the vision project | §5 → §6 → §7 → §8 → §9, with §11 as the interface contract |
| Writing the robot program | §12 → §13 → §14, with Appendix A/B/C on the bench |
| Supporting a running cell | §14 → §17 → §19, with Appendix A open |

### 0.2 Verification legend

Every technical claim in this handbook carries one of three confidence markers. Machine tending puts a robot arm inside a machine tool; the difference between "documented" and "we think so" matters.

| Marker | Meaning |
| --- | --- |
| [D] | Documented by Mech-Mind. Traceable to an official page (see Appendix H). |
| [E] | Engineering practice. Not in the manuals; standard integration practice, or derived from the documented behaviour. Safe, but verify against your build. |
| [?] | Unverified or build-specific. Named in Appendix F. Confirm on your machine before relying on it. |

### 0.3 Conventions

- Poses. Internally Mech-Vision carries object poses as quaternions — [x, y, z, qw, qx, qy, qz], 7 components [D]. On the wire to a robot, poses are always 6-DoF Euler: X Y Z in mm, A B C in degrees, in the Euler convention of the selected robot brand [D].

- Frames. Camera frame = the camera's optical frame. Robot frame = the robot base frame. World frame = the simulation origin in Mech-Viz / the path planning tool. Every pose in this handbook is annotated with its frame; unannotated poses in a real project are the number-one source of commissioning defects [E].

- Steps are written in Title Case exactly as they appear in the Step Library — Sort 3D Poses V2, not "sort poses".

- Commands are Standard Interface command numbers — 101, 102, 105.

- Vision point = an object recognised by Mech-Vision (pose, label, dimensions, custom data). Waypoint = a point the robot passes through on a planned path (pose, label, motion type) [D]. These are different payloads fetched by different commands; conflating them causes status 1016.

## 1. Machine tending as a vision problem

### 1.1 What makes it different from bin picking

Machine tending looks like bin picking with an extra place position. It is not. Four differences drive every design decision in this handbook:

|  | Bin picking (to conveyor/tote) | Machine tending (to chuck/vise/nest) |
| --- | --- | --- |
| Place tolerance | Loose — anywhere in the drop zone | Tight — the part must seat. A three-jaw chuck or a vise on a drill/tap machine wants sub-millimetre concentricity and a controlled orientation |
| Consequence of a bad pick | Part dropped, cycle retried | Part crashed into the fixture, jaws closed on air, spindle started on an unseated blank, tool broken |
| Cycle coupling | Robot is the only actor | Robot is a servant of the machine cycle — door open, chuck open, coolant off, air blast done, part unclamped |
| State | Stateless: pick next part | Stateful: is the nest occupied? Is this a load cycle or an unload cycle? |

Consequences you will see repeated later: orientation validation is mandatory, not optional (§7.5); the cell needs an explicit occupancy test (§9.2); and the robot program — not the vision project — owns the machine handshake (§2.3).

### 1.2 Cell archetypes

Pick the archetype first; it determines your camera mounting, your project count, and whether you need Mech-Viz.

| # | Archetype | Infeed | Vision job | Typical planner |
| --- | --- | --- | --- | --- |
| A1 | Single-station load only | Bulk bin or tote of blanks | Find one pickable blank per cycle | Path Planning step |
| A2 | Load + unload, single gripper | Bin in, finished-parts tray out | Find blank; confirm nest state | Path Planning step + robot logic |
| A3 | Load + unload, double gripper | Layer tray / kitted pallet | Find blank; unload finished part in the same trip | Mech-Viz |
| A4 | Multi-station / tombstone | Pallet of blanks | Find blank; select station; orientation-critical insertion | Mech-Viz |
| A5 | Mixed part numbers | Kitted tray, several SKUs | Recognise which part, then locate it | Either + parameter recipes (§16) |
| A6 | Orderly tray unload → machine | Layered blue-marked tray (as in the reference cell) | Locate every nest position, sequence them | Path Planning step + Sort 3D Poses V2 |

The reference Drill_Tap cell is an A6/A2 hybrid: parts stand in a dense grid fixture, the project recognises target object t1, validates orientation twice, sorts, trims, and hands the robot a planned path.

### 1.3 The tolerance budget

Before writing a single step, write the budget. Mech-Mind's own design guidance is the anchor:

- ±3–5 mm is enough for common projects [D]. Most machine tending is not common — it is at or below ±1 mm at the fixture.

- For ±1 mm class work, Mech-Mind directs you to deploy Vision System Drift Auto-Correction to counter temperature-driven error [D].

- Camera vision-guided picking accuracy (not metrology accuracy): NANO ULTRA-GL ±1 mm, PRO S-GL ±0.5–±1 mm, PRO M-GL ±1.5–±2 mm, LSR family ±1.5–±3 mm, DEEP-GL ±5 mm [D].

- Do not quote the VDI/VDE 2634 figure (0.1 mm @ 0.6 m for NANO ULTRA) as system accuracy. That is a metrology number for the sensor alone; the classic spec error is conflating it with picking accuracy [D].

A workable budget for a chuck-loading cell [E]:

| Contributor | Typical | Notes |
| --- | --- | --- |
| Camera picking accuracy | ±1.0 mm | Model-dependent, above |
| Hand-eye calibration residual | < 2.0 mm (high-precision acceptance band) | §6.5 |
| Robot repeatability | ±0.05–0.1 mm | From the robot datasheet |
| Gripper compliance / part slip in jaws | 0.2–1.0 mm | Measure it; do not assume |
| Fixture / chuck lead-in | −(chamfer) | This is your friend: a 2 mm lead-in chamfer buys 2 mm |
| Thermal drift over a shift | 0.3–1.0 mm | Mitigate with drift auto-correction [D] |

Design rule [E]: if the sum of the first four exceeds the fixture's lead-in, the cell will fail intermittently no matter how good the vision project is. Fix it mechanically — add a chamfer, a compliant gripper, a re-grip station, or a secondary positioning platform — before tuning software. Mech-Mind's own application templates route difficult loading through a "secondary platform" / "positioning platform" for exactly this reason [D].

### 1.4 What vision must deliver, in order of importance

- A pose that is graspable — not just detected. The pick point must be reachable with the actual gripper, without collision with bin walls, neighbouring parts, or the fixture.

- A pose whose orientation is loadable — a blank at 40° in a bin is unloadable into a chuck even if the pose is perfect (§7.5).

- Correct metadata alignment — the label, dimensions and pick-point info travelling with that pose must belong to that pose (§7.7). Misaligned metadata is how a robot loads the right position with the wrong gripper recipe.

- A deterministic answer to "nothing to pick" — an empty result must be distinguishable from a fault (§14).

- Repeatable order — for tray/pallet unloading, the sequence must be legible to an operator (§7.6).

## 2. System architecture

### 2.1 Components and who owns what

┌──────────────┐  GigE   ┌──────────────────────────────────────┐
   │ Mech-Eye 3D  ├────────►│  IPC (Windows)                       │
   │ camera       │         │  ├── Mech-Eye SDK / Viewer           │
   └──────────────┘         │  ├── Mech-Vision  (recognition)      │
                            │  │    └── Path Planning step (opt.)  │
                            │  ├── Mech-Viz     (motion, optional) │
                            │  └── Communication Component         │
                            │        (Standard Interface server)   │
                            └───────────────┬──────────────────────┘
                                            │ TCP (vision = server)
                                            ▼
   ┌────────────────────┐   I/O / fieldbus   ┌──────────────────┐
   │ Robot controller   │◄──────────────────►│ Machine tool /   │
   │  MM_* routines +   │                    │ CNC / PLC        │
   │  cell program      │                    │ (door, chuck,    │
   └────────────────────┘                    │  M-codes)        │
                                             └──────────────────┘

| Component | Owns | Does not own |
| --- | --- | --- |
| Mech-Eye camera | Raw 2D image, depth map, point cloud | Any notion of a robot |
| Mech-Vision | Recognition, pose processing, validation, sorting; optionally local path planning | The machine handshake; long moves |
| Mech-Viz (if used) | Full cycle motion logic, collision-checked paths, branch/counter/DO logic | Machine M-codes |
| Communication Component | The Standard Interface TCP server, protocol encode/decode, status codes | Any decision-making |
| Robot controller | Motion, safety, the machine handshake, error recovery, retry policy | Recognition |
| PLC / CNC | Door, chuck/vise, coolant, part-present sensors, cell interlocks | Poses |

Architectural rule [E]: the robot program is the cell's state machine. Vision answers questions; it does not run the cell. This single rule prevents the most common architecture mistake in machine tending — burying the door-open/chuck-clamp handshake inside a Mech-Viz workflow, where it becomes invisible to the maintenance electrician at 2 a.m.

### 2.2 Communication modes — choose once

|  | Master-Control | Standard Interface | Adapter |
| --- | --- | --- | --- |
| Who sends commands | Vision system → robot | External device (robot/PLC) → vision | External device → vision |
| Robot-side code | "No programs are needed" [D] | Programs written for the robot [D] | Programs on both sides [D] |
| Protocols | TCP, UDP | TCP, UDP, Snap7, PROFINET, EtherNet/IP, Modbus TCP, Mitsubishi MC [D] | All of the above + HTTP, WebSocket [D] |
| Difficulty / flexibility | Low / Low [D] | Medium / Medium [D] | High / High [D] |
| Workpiece loading supported | ✔ | ✔ | ✔ |
| Gluing supported | ✔ | ✘ [D] | ✔ |

Use Standard Interface for machine tending [E]. Reasons: the robot must own the machine handshake and safety (§2.1), the documented capability gap is gluing only [D], and the customer's maintenance team can read a robot program. Master-Control is attractive for demos and awkward in production because the vision PC becomes a motion authority. The reference cell has Robot Communication Configuration enabled — the toolbar toggle is on — which is exactly this mode.

### 2.3 Where the machine handshake lives

ROBOT PROGRAM (owner of the cycle)
  wait  DI[machine_cycle_complete]
  wait  DI[door_open]           ← from CNC via I/O or fieldbus
  ... unload finished part  (vision optional; often a taught position)
  set   DO[chuck_open]
  ... load new blank        (vision-guided: 101 → 102/105)
  set   DO[chuck_close]
  wait  DI[chuck_clamped]
  set   DO[start_cycle]         ← M-code / cycle start to the machine
  retreat clear of the door
  set   DO[door_close]

Vision is called only inside the "load new blank" bracket. Everything else is deterministic robot/PLC logic [E].

## 3. Hardware and physical design

### 3.1 Camera selection

For the reference hardware, the published specification [D]:

| Spec | NANO ULTRA-GL |
| --- | --- |
| Working distance | 350M variant: 250–500 mm (object focal 350 mm) · 700M variant: 400–800 mm (object focal 700 mm). These are two SKUs, not two ranges of one camera |
| FOV | 220 × 165 mm @ 0.25 m; 400 × 270 mm @ 0.4 m; 500 × 340 mm @ 0.5 m; 770 × 550 mm @ 0.8 m |
| Z repeatability (1σ) | 0.045 mm @ 0.4 m; 0.1 mm @ 0.6 m |
| VDI/VDE 2634 Pt. II | 0.1 mm @ 0.4 m and @ 0.6 m |
| Point cloud resolution | 2400 × 1800 (4.3 MP), reducible to 1200 × 900 |
| Capture time | 0.5–0.9 s |
| Light source | Blue LED, 440 nm, RG2 |
| Vision-guided picking accuracy | ±1 mm |
| IP / temperature / humidity | IP65 / 0–45 °C / 0–85 % RH non-condensing |
| Power | 24 VDC, 3.75 A |
| Size / weight | 125 × 46 × 76 mm / 0.7 kg |

Notes that matter in a real cell:

- Confirm which variant you have before designing the mount. NANO ULTRA is published in a 350M (250–500 mm) and a 700M (400–800 mm) variant [D]; the FOV and hence the usable bin footprint differ substantially. The reference unit's label reads only Model: NANO ULTRA with SN RUM70249B500E015, so read the variant from Mech-Eye Viewer or the delivery paperwork rather than the case label [?].

- The ±1 mm vision-guided picking accuracy figure comes from the Camera Selection Guide, not the specification sheet [D] — quote it as a selection-guide figure.

- Mech-Mind's model comparison lists NANO ULTRA-GL for "metal part loading/unloading and picking" and "high-accuracy assembly and screw-driving" — i.e. precision short range [D].

- Mech-Mind's machine-tending solution page recommends PRO S-GL / PRO M-GL / LSR L-GL [D]. If your standard is NANO ULTRA, be deliberate: it is the precision/short-range choice, and FOV at 0.5 m is only ~500 × 340 mm. A 600 × 400 mm pallet will not fit in one frame — plan multiple capture positions or a different camera.

- Mounting mode: the model-comparison table lists NANO ULTRA-GL for eye-in-hand [D]; LSR XL is ETH-only, other models support both [D]. Confirm ETH support for your exact SKU with Mech-Mind before designing a fixed-post mount [?].

### 3.2 Eye-to-hand vs eye-in-hand for machine tending

|  | Eye-to-hand (fixed post) | Eye-in-hand (on the flange) |
| --- | --- | --- |
| Cycle time | Better — capture can overlap robot motion; supports "capture-and-move" [D] | Worse — robot must travel to the capture pose |
| FOV coverage | Fixed. Large infeed needs a taller mount and a bigger camera | Excellent — fly to any region, including inside the machine |
| Calibration | ETH process; simpler ongoing | EIH process; more sensitive to TCP accuracy |
| Cabling | Static | Dress pack, flex cycles, a lifetime item |
| Machine-side inspection | Can't see into the machine | Can — useful for nest occupancy and chip detection |
| Collision risk | None (camera out of the work zone) | Camera becomes part of the tool envelope; model it in collision detection |

Default for machine tending [E]: eye-to-hand over the infeed, because cycle time is the commercial metric and the fixed post keeps the camera out of coolant and chips. Choose eye-in-hand when the cell must look inside the machine (nest occupancy, chip check) or when the infeed area exceeds one FOV.

### 3.3 IPC setup

The Mech-Mind IPC ships configured [D]; if you build your own, or reimage, these are the documented requirements:

| Item | Requirement |
| --- | --- |
| CPU | Must support the AVX2 instruction set [D] |
| GPU driver | Mech-Vision 1.7.0 / Mech-DLK 2.3 require NVIDIA 472.50+ [D] |
| Region & language | IPC region and language must match, or logs render garbled [D] |
| Firewall | Disable Windows Defender Firewall on the adapters facing camera / robot / PLC, across Domain, Private and Public profiles [D] |
| Windows Update | Disable both Windows Update and Windows Update Medic Service — the Medic service re-enables Update by itself [D] |
| License dongle | Plug full license dongles in before installing software ≥ 1.6.0 [D] |
| Non-deep-learning speed | Depends on CPU multi-core performance, not the GPU [D] |

IPC STD hardware facts worth having on the wiring drawing [D]:

- NIC ports are labelled CAMERA 1 / CAMERA 2 / CAMERA 3-ROBOT / ROBOT-PLC.

- Speed LED: off = 100 Mbps, orange = 1 Gbps, green = 2.5 Gbps. Camera links must not be at 100 Mbps — normal camera transmission is 700–800 Mbps [D], so an orange/green LED is a commissioning check.

- PoE is not supported — the camera needs its own 24 VDC.

- Digital I/O connector: DIN0–DIN5, DOUT0–DOUT5, +5VS, GND on pins 1 and 8.

- DB9 serial COM1–COM4 (RS232) and COM5–COM6 (RS232/422/485).

- Remote power connector: a short-circuit signal on the 2-pin terminal triggers a clean shutdown — wire it to a PLC output so the cell can power the IPC down gracefully on plant shutdown.

- Start-up beep codes are in the IPC manual's Appendix A; a cell that "won't boot" is often a memory seating fault with a documented beep pattern.

### 3.4 Network plan

| Link | Recommendation |
| --- | --- |
| Camera ↔ IPC | Dedicated NIC, gigabit or better, its own subnet. Never share with the plant LAN [E] |
| IPC ↔ robot | Dedicated NIC. IPC and controller in the same subnet, different addresses [D] (docs example: 192.168.100.169 / .170, mask 255.255.255.0) |
| IPC ↔ PLC | Separate NIC where available (the STD has ROBOT-PLC) |
| Plant LAN | Ideally not connected. If required for backups, treat as untrusted and keep Windows Update disabled [D] |

FANUC-specific: if both Port 1 (CD38A) and Port 2 (CD38B) are enabled, their IP addresses must be on different subnets [D].

Port number. There is no single hard default [D]:

- ABB / KUKA / YASKAWA setup pages recommend 50000 or above [D].

- Official FANUC examples use 30000, and on FANUC V10 the port range is 1–32767 — so 50000 is invalid there [D].

- TCP supports concurrent processing on a maximum of four ports; if multiple ports are used, modify the example program so global variables are not shared between ports [D].

### 3.5 The environment next to a machine tool

Machine tending puts a 3D camera in one of the worst optical environments in a plant. Design against it [E], with the documented levers noted:

| Hazard | Mitigation |
| --- | --- |
| Coolant mist, oil film on the lens | Air-purged enclosure or a sacrificial window; scheduled lens cleaning (the docs list regular lens cleaning as a design constraint [D]) |
| Chips on the infeed / in the nest | Air blast before capture; nest occupancy check (§9.2) |
| CNC work lights, strobing | Anti-Flicker Mode suppresses depth fluctuation from artificial lighting [D] |
| Ambient sunlight / high ambient light | LSR (laser) models have higher ambient-light immunity [D] |
| Vibration from the machine, VFD EMI | Rigid, decoupled camera post; separate cable routing; the docs call out vibration tolerance and EMI as design constraints [D] |
| Temperature swing over a shift | Drift Auto-Correction for ±1 mm class work [D]; camera rated 0–45 °C [D] |
| Camera post knocked by a forklift or an operator | Mechanical guard, and a daily calibration check in the shift start-up routine (§19.1) [E] |

### 3.6 Safety

## Out of scope for the software, but state it in every project file [E]: vision does not make a cell safe. The robot's safety-rated stop, the light curtain or fence interlock, the machine's door interlock, and the chuck-clamped confirmation are all safety-rated I/O owned by the robot/PLC. A vision result must never be a condition for a safe state — only for a motion target.

## 4. Bring-up sequence

Mech-Mind's own tutorial spine is the right order, and it is worth following literally because each phase's output is the next phase's input [D]:

Vision Solution Design → Vision System Hardware Setup → Robot Communication Setup → Hand-Eye Calibration → Workpiece Locating → Pick and Place

Expanded for machine tending [E]:

| Phase | Output | Gate before proceeding |
| --- | --- | --- |
| 0. Concept | Archetype (§1.2), tolerance budget (§1.3), camera + mount choice | Budget closes mechanically |
| 1. Hardware | Camera mounted, IPC configured, networks up, license dongle in | Camera visible in Mech-Eye Viewer; link at 1 Gbps+ |
| 2. Image quality | Capture parameter group tuned for the actual part finish | Point cloud complete on the worst-case part (§5.5) |
| 3. Communication | Robot Communication Configuration applied, MM_* programs loaded, comms test passes | MM_COMTEST (or brand equivalent) returns a good status |
| 4. Calibration | Hand-eye calibration saved, error within band | Error < 2 mm at 100 % for high-precision work (§6.5) |
| 5. Target object | Point cloud model + pick points + collision model in the target object editor | Recognition succeeds on 20 consecutive frames, worst-case presentation |
| 6. Vision project | Full pipeline (§7) | Correct pose, correct order, correct empty-result behaviour |
| 7. Motion | Path Planning workflow or Mech-Viz project, collision models real | Simulated cycle collision-free at production speed |
| 8. Robot program | Full cycle incl. machine handshake and error handling (§13, §14) | Dry cycle without a part; then with one part |
| 9. Commissioning | Acceptance protocol signed (§18) | Soak test passed |
| 10. Handover | Backups, documentation, operator training, spares | §17.5 |

Do not skip phase 2 or 4. A poorly exposed point cloud and a 4 mm calibration residual both present as "the recognition is unreliable", and both are invisible in the step graph.

## 5. Camera setup for machined metal

Machine-tending parts are the hard case: turned steel, ground shafts, black-oxide drill blanks, aluminium with a fresh cut face. All of them are specular; some are both dark and specular in the same frame.

### 5.1 Tune in Mech-Eye Viewer, use the group in Mech-Vision

Parameters live in a configuration parameter group in Mech-Eye Viewer; the Capture Images from Camera step then selects that group [D]. Never tune inside the vision project — tune in Viewer, save the group with a meaningful name (t1_blank_reflective), and select it in the step. That way the same group can be reused by other projects and audited [E].

### 5.2 3D exposure — the documented method

Single-material part [D]:

- Set Exposure Multiplier = 1, Flash exposure mode, Fast acquisition.

- Capture once and judge the depth-source 2D image: too dark → increase Exposure Time; too bright → decrease.

- Missing depth-map data indicates underexposure.

Mixed or shiny surfaces — the normal machine-tending case [D]:

- Find the exposure time that images the brightest/most reflective region correctly.

- Find the exposure time that images the darkest region correctly.

- Set Exposure Multiplier = 2 and enter both values in Exposure Time and Exposure Time 2.

- If coverage is still incomplete, add a third exposure and set the multiplier to 3.

Cycle-time alternative [D]: Fringe Coding Mode = Reflective gives single-pass acquisition for mixed/shiny surfaces, and is available on DEEP-GL, LSR, NANO ULTRA-GL and PRO. On a reflective-part cell this preserves takt time versus stacking two or three exposures — try it before you accept a 3× capture penalty.

### 5.3 Other levers for reflective parts

| Parameter | Setting for shiny metal | Source |
| --- | --- | --- |
| Light Brightness (LED projector) | Low for reflective objects (High for dark, Normal for regular) | [D] |
| Laser Power (laser models) | Lower intensity for highly reflective objects; higher for dark | [D] |
| Fringe Coding Mode | Reflective (mixed/shiny); otherwise Fast vs Accurate trade-off | [D] |
| Anti-Flicker Mode | On, near CNC work lights | [D] |
| Gain | Use sparingly — raises brightness but adds noise | [D] |

Per-model parameter defaults (Gap Filling, Fringe Contrast Threshold, Outlier Removal, Surface Smoothing, Distortion Correction, Anti-Blur) live in the camera manual's per-model Parameter Reference Guide chapter; pull exact values from there for your SKU [?].

### 5.4 Depth range and 3D ROI — also a cycle-time lever

Three parameters need Mech-Eye Viewer's visual tools and several rounds of fine tuning [D]:

- Auto-Exposure ROI (2D parameters, when exposure mode = Auto).

- Depth Range — upper and lower limit.

- 3D ROI — x, y, width, height.

Trim both to the actual working volume. It removes bin walls and machine structure from the cloud and cuts data volume and processing time — the documented capture-side cycle-time levers are "minimise exposure time, update firmware, trim ROI and depth range, gigabit network" [D].

Machine tending caution [E]: leave 20–30 mm of margin around the bin/pallet footprint for placement variation, and re-check the depth range when the tray is empty — a range tuned on a full tray can clip the bottom layer.

### 5.5 Image-quality acceptance test

Before moving on, prove the capture [E]:

- ☐ Worst-case part (darkest, shiniest, most tilted) has a complete point cloud on the graspable surface.

- ☐ Depth holes, if any, are away from the pick point and the matching features.

- ☐ Point cloud fluctuation across 10 static captures ≤ 3 mm (the documented acceptance limit for calibration-time fluctuation, a reasonable bar here too) [D].

- ☐ Full tray and near-empty tray both image acceptably.

- ☐ Capture time recorded, and it fits the takt budget (§15).

- ☐ Parameter group saved with a versioned name and backed up.

## 6. Hand-eye calibration

### 6.1 Choose the process

The calibration guide branches on four axes [D]: robot type (6-axis / 4-axis SCARA & palletizer / 5-axis / gantry), communication mode (Standard Interface or Master-Control), camera mounting mode (ETH/EIH), and data-collection method (Automatic / Manual TCP-touch / Manual random board poses).

For a typical machine-tending cell: 6-axis, Standard Interface, eye-to-hand, Automatic.

### 6.2 Automatic ETH calibration — documented workflow

- Prerequisites [D]: full system installed; calibration board on a robot-specific bracket at the flange; robot parked at the bottom centre of the camera FOV; 2D/3D parameters tuned in Mech-Eye Viewer so the board images cleanly; robot communication configured.

- Mech-Vision → Camera Calibration → New calibration → Hand-eye calibration for listed robot → robot model → Eye to hand → Automatic → Standard Interface: set protocol and host IP, start the interface service, and run the auto-calibration program on the teach pendant [D].

- Select board type Standard, matching the model on the board's nameplate; place the board at the centre of the FOV (red rectangle) [D].

- Check intrinsic parameters (Start checking). If circle detection is poor, draw aid circles or hand-tune the detection thresholds [D].

- Configure the pyramid calibration path: Height span, Path type = ToHand, Num of layers, Bottom/Top-layer dimensions X/Y, Motion grid cols and rows per layer, Rotation angle [D].

- Tick Save images and run Auto move robot along path and capture images [D].

- Calculate extrinsics, inspect the error, save [D].

The robot side of this handshake is command 701: the robot sends 701, calibration status, flange pose, joint positions where sent-status 0 = "initiate calibration", 1 = "reached previous point", 2 = "failed to reach"; the reply carries the next calibration point and a returned status of 0 = in progress, 1 = finished [D]. Each brand provides an MM_CALIB routine and a ready-made auto-calibration program (§13).

### 6.3 Manual calibration (TCP touch) — when automatic is impossible

Needed when the robot cannot carry the board, or the cell geometry blocks the pyramid path [E]. Documented essentials [D]:

- A sharp tip on the flange, an undamaged board with clear circles, board placed at the centre of the workobject plane.

- Wizard: New calibration → Hand-eye calibration for custom robot + Euler-angle convention → robot type → Eye to hand → TCP touch → connect camera → mount/place board → intrinsic check → Set TCP in flange frame.

- At least three points not in a line must be touched to compute extrinsics.

- Touch the cross centre of each point, read the flange pose from the pendant, type it in; then capture images with Save images on.

- Use Calculate rotation and translation separately; review the error in the point cloud viewer.

### 6.4 Calibration boards

| Family | Models (circle spacing) | Grid | Notes |
| --- | --- | --- | --- |
| BDB | BDB-5 (20 mm), BDB-6 (35 mm), BDB-7 (50 mm) | 5×4 | V2/V3 camera generations [D] |
| CGB | CGB-020 (20 mm), CGB-035 (35 mm), CGB-050 (50 mm) | 5×4 | V4 cameras — current [D] |
| OCB | OCB-005 (5 mm), OCB-010 (10 mm), OCB-020 (20 mm) | 9×11 | EIH only — no mounting holes, cannot be bolted to an end effector [D] |

Boards serve two purposes: the intrinsic parameter check and the hand-eye calibration itself [D]. For NANO ULTRA at 250–800 mm expect a CGB-020 or CGB-035 class board — confirm against the Calibration Board Selection chapter before ordering [?].

### 6.5 Acceptance — the numbers that matter

Published acceptance thresholds, all points must pass [D]:

| Project class | Threshold |
| --- | --- |
| Standard projects | error < 3 mm (100 %) |
| High-precision projects | error < 2 mm (100 %) |
| (De)palletizing | error < 5 mm (100 %) |

Machine tending into a chuck or vise is a high-precision project — hold the 2 mm band [E].

Checks [D]:

- Point Cloud Viewer error map — darker = larger error; number keys 0–9 isolate error bands in 0.5 mm increments; quote the value at the 100 % percentile.

- ETH: the robot's point cloud should roughly coincide with the robot model. EIH: the board's point cloud must stay stable across poses.

- Euler-angle consistency: angles should fluctuate < 1° along the pyramid path. More than that suggests the robot has lost its zero position — stop and fix the robot, not the calibration.

- Intrinsic check: watch "Error of measured average calibration circle interval"; out-of-spec points turn yellow.

- Point cloud fluctuation: max acceptable 3 mm. Exceeding it at several points means an exposure problem or mechanical instability — fix before recalculating.

- Remedies: optimise exposure, clamp the board rigidly, reduce robot speed, recompute compensation parameters.

### 6.6 How many poses

- Manual/TCP-touch: ≥ 3 non-collinear points [D].

- More is not better: "Too many calibration points may introduce abnormal points, leading to an increase in the overall error ratio." Recommended grid by focal distance: 2×2 for 300–2000 mm; 3×3 for 2000–3500 mm [D]. A NANO ULTRA cell sits in the first band → 2×2 per layer.

### 6.7 Drift, camera swaps, and when to recalibrate

- Mech-Mind ships Vision System Drift Auto-Correction (with ETH and EIH deployment guides and a calibration-sphere hardware guide) specifically for temperature-driven error, and recommends it for ±1 mm class accuracy [D].

- The Production Interface can monitor accuracy drift in production [D] — enable it; it turns a slow failure into a visible trend.

- A Quick Camera Replacement feature exists for swapping a camera without a full recalibration [D]; its prerequisites are not documented on the pages reachable for this handbook [?].

- There is no official statement requiring recalibration after the camera is bumped [?]. Treat "camera or camera post moved, robot repaired, or robot zeroed → redo hand-eye calibration" as mandatory engineering practice [E], and put a witness mark on the camera bracket so a bump is visible [E].

## 7. The vision project

### 7.1 The canonical machine-tending pipeline

Every machine-tending vision project reduces to seven functional stages. The reference Drill_Tap project implements all of them.

[1] ACQUIRE      Capture Images from Camera
        │
 [2] RECOGNISE    3D Target Object Recognition            (target object "t1")
        │              ├─► Pick Points
        │              └─► Pick Point Info (+ optional ports)
        │
 [3] GATE          Validate Poses by Included Angles to Reference Direction   (tilt)
                   Validate Existence of Poses in 3D ROI                       (reach / nest)
        │              └─► Validation Results (Bool[])
 [4] CULL          Filter   ×N ports  ← driven by the Bool[] above
        │
 [5] ORDER         Sort 3D Poses V2  ─► Mapped Indices ─► Reorder by Index List
        │
 [6] LIMIT         Trim Input List   (Output Size = parts per cycle)
        │
 [7] NORMALISE     Flip Poses' Axes → Transform Poses
        │
 [8] PLAN          Path Planning     (or hand off to Mech-Viz)
        │
 [9] EMIT          Output            (the interface contract, §11)

 Parallel:         Show Point Clouds and Poses    (diagnostic tap, no data output)
                   Trigger Control by Flag        (per-cycle branching, §9)
                   Easy Create Poses              (constant reference poses)

Stages 3–6 are the ones that turn "detected" into "loadable", and they are where most projects are under-built.

### 7.2 The reference project, step by step

Read off the Drill_Tap step graph (Mech-Vision 2.1.1, 37 steps, ~1.50 s execution):

| # | Step (as named in the project) | Role in the pipeline | Handbook § |
| --- | --- | --- | --- |
| 1 | Capture Images from Camera (1) | Acquire depth map + colour image | §8.1 |
| 2 | 3D Target Object Recognition (1) — target object t1, Config wizard | Recognise blanks, emit pick points | §8.2 |
| 3 | Easy Create Poses (1) / (2) | Constant reference poses for the two angle gates | §8.3 |
| 4 | Validate Poses by Included Angles to Reference Direction (1) | First tilt gate (pre-normalisation) | §8.4 |
| 5 | Validate Existence of Poses in 3D ROI (1) | Reach / region gate | §8.5 |
| 6 | Sort 3D Poses V2 (1) / (2) | Two-key ordering (e.g. layer then row) | §8.6 |
| 7 | Filter (1)–(4) | Cull all parallel lists using the Bool[] verdicts | §8.7 |
| 8 | Reorder by Index List (1) / (2) | Keep metadata aligned with the sorted poses | §8.8 |
| 9 | Trim Input List (1) / (2) | Keep only the parts this cycle will use | §8.9 |
| 10 | Validate Poses by Included Angles to Reference Direction (2) | Second angle gate, after normalisation | §8.4 |
| 11 | Trigger Control by Flag (1) | Per-cycle branch | §8.10 |
| 12 | Flip Poses' Axes (1) | Force the approach axis to point into the part | §8.11 |
| 13 | Transform Poses (1) | Camera → robot frame (or fixture-relative) | §8.12 |
| 14 | Path Planning (1) — Config wizard | Local collision-free approach/pick/depart | §8.13 |
| 15 | Output (1) | Emit to the Standard Interface | §8.14 / §11 |
| — | Show Point Clouds and Poses (1) | Diagnostic visualisation | §8.15 |

Why the double gate (steps 4 and 10) is correct [E]. Gate 1 removes grossly mis-oriented detections before any pose surgery, which keeps the downstream lists small and the sort meaningful. Gate 2 runs after Flip Poses' Axes normalises the approach axis, so it measures the tilt the gripper will actually see. A single gate placed before the flip will reject good parts whose Z happens to point up (they read ~170° off); a single gate placed after the flip will let through parts that were mis-detected rather than merely flipped.

Why the paired sorts and reorders [E]. Sort 3D Poses V2 emits Sorted Poses and Mapped Indices. Two sorts in series give a compound key — for a layered tray, sort by Z (layer) then S-shape on plane (row/column within the layer). Each sort's indices must be applied to every parallel list via Reorder by Index List, or the labels stop belonging to the poses.

### 7.3 Project hygiene

Conventions that pay for themselves on the second visit [E]:

- Name every step instance for its job, not its type: Gate_Tilt_PreFlip, Sort_Layer_Z, Trim_OnePerCycle. The default (1), (2) suffixes are how a 37-step project becomes unmaintainable.

- Comment the frame on every step that touches poses. Write "camera frame in, robot frame out" on Transform Poses.

- Keep a Show Point Clouds and Poses tap at the end of the pose chain with its eye icon off. It costs nothing in production and saves an hour when a part mis-picks [D] (the step has no data output, so it cannot affect the result).

- One Output step per project — that is a hard limit [D]. Multi-machine cells are multiple projects in one solution.

- Version the solution folder before every change (§17.4).

- Record the step count and execution time of a known-good build (Drill_Tap: 37 steps, 1.50 s). It is the cheapest regression test you have [E].

### 7.4 Recognition: target object, model, pick points

3D Target Object Recognition integrates point-cloud preprocessing, deep learning and removal of overlapped objects, and its documented usage scenario is workpiece loading — separate arrangements, orderly single-layer, orderly multi-layer, and random stacking [D]. That is machine tending, exactly.

The target object editor consolidates point-cloud model generation/editing, symmetry settings, object centre point, pick points, and the collision model, through six guided workflows [D]. Key concepts [D]:

- A point cloud model is a predefined cloud reflecting the object's shape and features, matched against the scene cloud. Requirements: enough evenly distributed points (speed), typical features present (accuracy), no interfering points (stability).

- A pick point is a grippable position defined in the object reference frame. An object may have several, inside, on, or near the cloud.

- Models can be generated from STL CAD by setting units and sampling the exterior surface [D].

- Compatibility: target objects configured with 2.1.2 cannot be used with software earlier than 2.1.2 [D]. Pin your version.

- In the Target object selection and recognition view the visualisation shows object centre points; pick points appear only in the General settings view. Close the tool before an external service triggers the project, or results will not refresh [D].

Machine-tending practice [E]:

- Model the blank as it actually arrives — with the flash, the sprue, the sawn face. A pristine CAD model matches badly against a real casting.

- Define pick points that are chuck-compatible: grip on a surface that will not be the clamping surface, and leave the datum face clear.

- Give rotationally symmetric parts several pick points (clock positions) so the planner has options; then let symmetry settings and pick-point filtering choose.

- Enable Target object confidences and Overlap ratios output ports so sorting can prefer clean, unoccluded, top-layer parts [D].

- Enable Point clouds of matched target objects when you want post-pick error proofing (wrong part / short part) before the door closes [D].

Confidence. There is no parameter literally named "Target Object Confidence" [?]. What exists: the Target object confidences output port, and the underlying 3D Matching → Confidence Settings — Result Verification Degree (Low / Standard / High / Ultra-high, default Standard) and Confidence Threshold (default 0.3000), where confidence is defined as the coincidence ratio between the point cloud model and the scene point cloud [D]. Raise the threshold when you get false recognitions; lower it when you get false negatives. Deep-learning assist has its own separate confidence threshold, supports instance segmentation and object detection only, runs at batch size 1, and offers Dilation / Kernel size to stop undersized masks clipping edge cloud [D].

The optional output ports are added in the wizard under General settings → Configure output port [D]. Note that the port names visible on a step block in a given build may not match the documented checkbox names; the documented set is: Port(s) related to pick point, Port(s) related to object centre point, Pick point numeric labels, Target object confidences, Original point cloud acquired by camera, Preprocessed point cloud, Point clouds of matched target objects, Overlap ratios, Deep learning visualization result (which requires clicking Save, or the port is not added) [D]. Names such as "Pick Point IDs", "Colored Point Cloud", "Merged Point Cloud", "Object Point Clouds" seen on step blocks map onto these but are not documented under those names [?] — record the mapping for your build in your project notes.

### 7.5 The orientation gate — why machine tending needs it

A blank lying at 35° in a bin may be recognised perfectly and still be unloadable into a three-jaw chuck, and a tilted approach risks a gripper-to-bin-wall collision.

Configure Validate Poses by Included Angles to Reference Direction [D]:

| Parameter | Machine-tending setting |
| --- | --- |
| Axis to Be Specified | Z — the pick approach axis |
| Reference Direction | Positive/Negative Z of the world frame, per your convention (default Positive Z) |
| Reference Direction Source | Pose (fed from Easy Create Poses) or Vector3d |
| Max Angle Difference | The tilt the gripper and the fixture genuinely tolerate — commonly 15–30° for a two-finger or magnetic gripper feeding a lathe (default is 90°, which is far too permissive) [E] |

Wire Validation Results into a Filter so the whole parallel data set is culled consistently, not only the pose list. In 2.2.x the step also emits Angles (Number[], degrees) — log it as a process variable to watch tilt distribution drift as the tray empties [D]/[E].

Related steps, so you pick the right one [D]:

| Step | Use |
| --- | --- |
| Validate Poses by Included Angles to Reference Direction | The canonical angle gate |
| Calc Included Angles between Specified Axes of Poses | Compute an angle between two pose lists (Advanced Step) |
| Validate Pose Angle and Position of Target Objects | Validates angular and positional deviation vs a reference — "scenarios requiring strict control over the position and orientation of target objects", i.e. assembly-grade insertion |
| Adjust Poses V2 → Filter Poses by Predefined Options → Filter by angle | The same function inside the pose-adjustment tool |
| Rotate Axis to Minimize Included Angle to Reference Direction | Rotation, not validation — do not confuse it |

### 7.6 Ordering: sorting is the picking strategy

Sort 3D Poses V2 offers eleven rules [D]. The ones that matter here:

| Rule | Machine-tending use |
| --- | --- |
| S shape on plane / Z shape on plane | Orderly single- or multi-layer trays and pallets. Documented use: "unloading neatly arranged workpieces or depalletizing". Parameters: Row direction (default +X), Column direction (default +Y), Row Interval (default 100 mm) |
| Z value of pose | Random bins — work top-down |
| Pose confidence values | Attempt the cleanest, least-occluded part first; pairs with Overlap ratios |
| Distance from pose to reference pose (or on XOY) | Minimise travel from the current position; pairs with Easy Create Poses as the reference |
| Projection of pose in reference direction | Unload along a defined axis, e.g. front-to-back in a fixture |

Align the row direction with the tray's long axis so the order is legible to an operator, and so robot travel stays monotonic and takt time stable [E].

### 7.7 Metadata alignment — the defect that hides longest

Sort 3D Poses V2 emits Mapped Indices; Reorder by Index List applies them [D]. Set Port Number to the number of parallel lists (up to 15) and drive them all from one Indices input. 2.2.1 states the intent explicitly: "suitable for scenarios where multiple relevant data (such as pick points, pick point information, and numeric labels of pick points) need to be sorted simultaneously" [D].

The same applies to Filter (multi-port) and Trim Input List (multi-port). Rule [E]: any step that changes the order or length of the pose list must change every parallel list identically, in the same step instance. Sorting poses without reordering their metadata is how a robot picks part A using part B's label — and it will pass every bench test, because on the bench the lists happen to be in the same order.

### 7.8 Normalisation before planning

Flip Poses' Axes forces the approach axis into the part: Axis to Flip = Z, Direction Type per your convention, Reference Axis to Rotate Around = X (it must differ from the axis being flipped) [D]. Matching returns a pose consistent with the point-cloud model, so a rotationally ambiguous blank — a symmetric flange, a shaft, a plate — can come back with Z pointing out of the bin, which the robot cannot approach. Normalising here reduces "unreachable pose" and near-singularity planning failures, and it is what makes the second angle gate meaningful [E].

Transform Poses then changes frame [D]:

| Transformation Type | Use |
| --- | --- |
| CameraToRobot (default) | The standard frame change every project needs |
| RobotToCamera | Inverse |
| AllWithFirst | The first reference pose transforms all original poses — express blanks in a measured fixture frame |
| FirstWithAll | All reference poses applied to the first original pose |
| UseCorrespondenceInput | Pairwise — per-station origins on a multi-station tombstone |

Always confirm what the next step expects: Validate Existence of Poses in 3D ROI has an explicit Input Poses Coordinate Type, and Path Planning/Output have a Point Cloud in Camera Frame checkbox for the same reason [D].

### 7.9 Limiting the result

## A machine tool consumes one part per cycle, and planning cost grows with the number of vision points evaluated. Put Trim Input List after sorting with Output Size = 1 for single load/unload, or = the number of parts a multi-gripper carries [D]/[E]. This also reconciles with the protocol: command 101 carries an expected number of vision points, and trimming keeps Mech-Vision's answer inside what the robot program is written to consume [D].

## 8. Step reference (machine-tending subset)

Version note: categories and some names changed between 2.1.x and 2.2.x [D]. Pin your handbook to one version and say which. This section is written for 2.1.x, with 2.2.x deltas marked ⚠.

| Step | Category 2.1.2 | Category 2.2.1 |
| --- | --- | --- |
| Capture Images from Camera | Capture | Data Acquisition |
| 3D Target Object Recognition | Recognition | Recognition |
| Easy Create Poses | Data Processing | Tools |
| Validate Poses by Included Angles to Reference Direction | Evaluation | Evaluation |
| Validate Existence of Poses in 3D ROI | Evaluation | Evaluation |
| Sort 3D Poses V2 | Pose Processing | Pose Processing |
| Filter | Evaluation | Evaluation, renamed Filter Data by Boolean Value |
| Reorder by Index List | Data Processing | Data Processing |
| Trim Input List | Data Processing | Data Processing |
| Trigger Control by Flag | Evaluation | Tools |
| Flip Poses' Axes | Pose Processing | Pose Processing |
| Transform Poses | Pose Processing | Pose Processing |
| Path Planning | Path Planning | Path Planning |
| Output | Transmission | Transmission |
| Show Point Clouds and Poses | Pose Processing | Pose Processing |

All of the above [D]. Note the common misfiling: Filter is under Evaluation, not Data Processing.

### 8.1 Capture Images from Camera

Outputs [D]: Camera Depth Map (Image/Depth), Camera Color Image (Image/Color), Point Cloud (PointCloud/XYZ), Colored Point Cloud (PointCloud/XYZRGB). A Color Image Path port also exists (implied by Image Name Type).

Key parameters [D]:

| Parameter | Machine-tending setting |
| --- | --- |
| Virtual Mode | Off in production; on for offline reproduction of a mis-pick from saved data |
| Camera ID / Select camera | The production camera |
| Calibration Parameter Group | Must be the group produced by the current hand-eye calibration, or every downstream frame transform is wrong |
| Configuration Parameter Group | The Mech-Eye Viewer group tuned in §5 (e.g. reflective) |
| Timeout | Default 10000 ms — covers connect and capture |
| Num of Reconnection Attempts / Max Capture Attempts | Default 3 each (3 recommended). This pair is the anti-nuisance-fault setting: a machine cycle should not hard-fault on one dropped frame |
| 2D Image Type / Rectify to Depth Map | LSR and DEEP-GL only; leave default otherwise |
| Virtual mode: Data Path / Play Back Mode | Folder of rgb_image_*.jpg + depth_image_*.png + intrinsics; Sequential / Repeat one / Repeat all / Random. MRAW files cannot be read |

The textured point cloud from this step is by default the project's scene point cloud, so images are still captured even if the step's cloud output is unconnected [D].

### 8.2 3D Target Object Recognition

Documented ports [D]: in — Camera Depth Map, Camera Color Image; out — Pick Points (Pose[]), Pick Point Info (JsonValue). Optional ports are added in the wizard (§7.4).

Step parameters [D]: Config wizard (also a button on the block), Select Target Object (the drop-down showing t1 in the reference project), and the execution flag Trigger Control Flow Given No Output — when selected the control flow still fires with empty output, and the step can still output the scene point cloud; Trigger Control Flow Given Output always takes effect.

The tool has three processes [D]: point cloud preprocessing (valid recognition region, edge detection, filtering) → target object selection and recognition (optional deep-learning assist, matching parameters) → general settings (output ports only).

### 8.3 Easy Create Poses

No input; output Pose (Pose[]) [D] ⚠ 2.2.1 port table. Parameter Poses: each pose is 7 components [x, y, z, qw, qx, qy, qz], default [0,0,0,1,0,0,0], multiple poses separated by semicolons [D].

Use for any constant, unsensed pose: the reference pose for an angle gate, a taught chuck-face or vise-jaw reference, the reference pose for distance sorting, or a hard-coded fallback drop pose during bring-up [E]. Always record which frame the literal is expressed in — this is the single most common source of confusion in load/unload projects [E].

### 8.4 Validate Poses by Included Angles to Reference Direction

Ports [D]: in Poses (Pose[]), optional Reference Poses (Pose[]), optional Reference Directions (Vector3D[]); out Validation Results (Bool[], same size as input), Valid Poses (Pose[]), ⚠ 2.2.x also Angles (Number[], degrees).

Parameters [D]: Reference Frame (display only: World frame), Reference Direction Source (Pose default / Vector3d), Reference Direction (±X/±Y/±Z or Customized; default Positive Z), Axis to Be Specified (X/Y/Z default), Max Angle Difference (0–180, default 90°), Output Empty List if Containing Invalid Poses, plus visualisation options. Angle ≤ threshold → True.

⚠ Parameter table is 2.2.1 text; the 2.1.2 page is marked "under maintenance" — verify against your build [?].

### 8.5 Validate Existence of Poses in 3D ROI

Ports [D]: in Poses; out Validation Results (Bool[]), Validated Poses (Pose[]).

Parameters [D]: 3D ROI Name (with Open the editor), Input Poses Coordinate Type (Robot frame / Camera frame), Show Poses Type. ⚠ 2.2.1 text [?].

Two machine-tending uses [E]: (1) a reach/keep-in gate on the infeed — a pallet corner that drifts outside the usable envelope, or a part that has fallen behind the bin, yields a valid pose the robot must never attempt; (2) an occupancy check on the machine side — draw the ROI around the chuck/vise/nest and use Validation Results to answer "is there a finished part to unload?" or "is the nest clear?", then branch with Trigger Control by Flag (§9.2). Watch the frame parameter: an ROI drawn in the robot frame silently rejects everything if poses arrive in the camera frame.

### 8.6 Sort 3D Poses V2

Ports [D]: in Poses; out Sorted Poses (Pose[]), Mapped Indices (Index[]).

Eleven rules [D]: None; S shape on plane; Z shape on plane; X / Y / Z value of pose; Distance from pose to reference pose; Distance from pose to reference pose on XOY plane; Pose confidence values; Projection of pose in reference direction; Cuboid diagonal lengths. Shape rules take Row direction (default +X), Column direction (default +Y), Row Interval (default 100.000 mm); value/distance rules take Ascending (default on); projection takes a Reference direction (default +X).

### 8.7 Filter (2.1.x) / Filter Data by Boolean Value (2.2.x)

Ports [D]: in Filter Criteria (Bool[]) + Unnamed (Abstract[]); out Unnamed (Abstract[]). The unnamed ports are literal — the step is type-agnostic, so only the Boolean port is named.

Parameters [D]:

| Parameter | Notes |
| --- | --- |
| Port Number (1–15) | Default 1. Set to 3 or 4 so one Boolean list culls pick points, pick point info, labels and dimensions together |
| Operation Layer (0–14) | Default 0 = operate on every element |
| Polarity | 2.1.x: Reverse Bool List (unselected = elements whose Boolean is False are filtered out). ⚠ 2.2.x renamed Keep Only Data with Boolean Value as False, default still unselected |
| Top Level | 2.2.x only, default unselected |

Usual predecessors: steps that output Bool[] — Dichotomize Values by Threshold (2.1.x) / Compare Value with Threshold (2.2.x), Validate Labels and Output Flags, the two Validate steps above, Validate Point Clouds [D].

Upgrade trap [D]: the polarity parameter was renamed between 2.1.x and 2.2.x while the default stayed "keep the True items". Re-read this checkbox after any upgrade.

### 8.8 Reorder by Index List

Ports [D]: in Indices (Index[]) + Unnamed (Abstract[]); out Unnamed. Worked example: indices 0,2,3,1 applied to 50,60,70,80 → 50,70,80,60.

⚠ 2.2.1 parameters [D]: Port Number (1–15), Operation Layer (0 = outermost), Top Level. 2.1.2 documents no parameters.

### 8.9 Trim Input List

Ports [D]: in Unnamed (Abstract[]); out Unnamed, same type. Parameters: Port Number (1–15), Operation Layer, Output Size (default 1) — the number of leading items kept.

### 8.10 Trigger Control by Flag

Ports [D]: in Boolean List (Bool[]) — the list should contain only one Boolean value; no data output — it emits a control-flow signal. Parameter ⚠ 2.2.1: Trigger Type = TriggerWhenTrue / TriggerWhenFalse [D].

Use for per-cycle logic (nest occupied → unload branch; zero parts → bin-empty branch), and Filter for per-part logic [E]. Feed it a one-element list, e.g. from a threshold comparison on a count — not a per-part Boolean list.

### 8.11 Flip Poses' Axes

Ports [D]: Poses in, Poses out. Parameters: Axis to Flip (X/Y/Z default), Direction Type (Negative default / Positive — the target direction of the flipped axis), Reference Axis to Rotate Around (X default / Y / Z, must differ from the axis to flip), Pose Type to Visualize.

Documented behaviour example: with Axis to Flip = Z and Direction Type = Positive, a pose whose Z axis is < 90° from the positive world axis is untouched, while one > 90° is flipped 180° around the reference axis [D].

### 8.12 Transform Poses

Ports [D]: in Original Poses, optional Reference Poses (required for AllWithFirst, FirstWithAll, UseCorrespondenceInput); out Transformed Poses. Transformation types as listed in §7.8. For truss/gantry robots use Transform Poses for Truss instead [D].

### 8.13 Path Planning (Mech-Vision step)

Requires Mech-Viz installed and licensed — the feature is Mech-Viz-related [D].

Documented usage: projects using Standard Interface or Adapter communication where only the robot motion near the vision point needs planning; after building a scene and inputting vision points, the step outputs a collision-free path after point-cloud collision detection [D]. Predecessor: pose processing. Successor: the Output step with Port Type = Predefined (robot path) (2.1.x) / Output Type = Robot path for picking (2.2.x) [D].

Ports — 2.1.2 [D]: in Vision Points, Collision Point Cloud, Pose Labels, Object Dimensions, plus Scene Object Names/Dimensions/Poses when Update Scene Object is enabled; out Planned Path, Filter Results (Bool[]: True = meets requirement). ⚠ 2.2.1 renames/expands [D]: in Pick Points (Pose[]), Pick Point Info (JsonValue), Collision Point Cloud (PointCloud[]), Pick Point Labels (String[]), Target Object Dimensions (Size3D[]), Pick Point Offset (Pose[]), scene-object ports; out Planned Path (JsonValue), Filtered Results (Bool[]). There is no documented output named "Error Results" — if your build shows that label, it is the per-vision-point pass/fail Boolean list [?].

Parameters [D]:

| Parameter | Machine-tending guidance |
| --- | --- |
| Workflow Configuration | Opens the path planning tool; select the configured workflow |
| Update Scene Object | Enable when the bin/pallet pose is measured each cycle, so collision checking uses the real pose |
| Select Scenario | 2.1.2: Matching (Machine Tending/Positioning/Assembly) (default) — your case, named explicitly. 2.2.1: 3D model matching (Machine Tending/Positioning/Assembly) |
| Method to Convert Data | Generate picking strategy based on object center point (default; symmetric objects — the step converts centre point to pick point) or based on pick points (multiple pick points needing filtering; you must feed pick point + pick point info) |
| Point Cloud Type | CloudXYZRGB (default) / CloudXYZ / CloudXYZNormal |
| Point Cloud in Camera Frame | Selected by default → the cloud is converted to the robot frame before being sent to the tool |
| Remove Point Cloud of Irregular Shape + Search Radius (default 3 mm) | Enable with a configured collision model when you need collision detection on the held object from pick to place — e.g. a long shaft on its way out through the machine door. Invalid for objects generated from common 3D shapes |
| Other Inputs / Other Ports | Adds Pick Point Labels, Target Object Dimensions, Pick Point Offsets. If enabled they must receive data, or the Output step fails |

The tool plans "a collision-free motion path near the target object"; its usage and some parameters differ from Mech-Viz, and Mech-Mind explicitly warns to consult each manual separately [D]. You choose the robot model from the Robot Model Library, then work through: configure project resources (tools, target objects, scene objects) → configure workflow → configure collision detection → simulate and debug [D].

Its default 6-step workflow is a ready-made machine-tending approach pattern [D]: Set Robot Tool → Above-Bin Fixed Waypoint 1 → Above-Workobject Approach Waypoint (relative move) → Vision Point → Above-Workobject Departure Waypoint (relative move) → Above-Bin Fixed Waypoint 2. Add further Fixed-Point Move / Relative Move steps as needed.

Collision detection [D]: by default collisions between any two of robot, robot tool, and scene objects are detected. Enabling Enable collision detection with point cloud adds tool-vs-cloud checking, with: point cloud form — Point cloud cube (fills the cloud surface with cubes) or Point cloud column (fills the space beneath the cloud with columns along −Z, extent set by Column base position); tool–point cloud collision volume upper limit; and point cloud collision record — Record highlights colliding regions in plan history but slows execution, Do not record is faster but untraceable.

Practice [E]: use Point cloud column when parts sit on an opaque pallet the camera cannot see through — otherwise the planner treats the space under the visible surface as free. Keep collision recording off in production, on only while chasing a specific collision report.

### 8.14 Output

Sends the vision result of the current project to Mech-Viz or the communication component. Only one Output step per project. It has no output ports. Hard rule: "All default ports must be connected to valid input data; otherwise, an error will occur when running the Output Step" [D]. Full contract in §11.

### 8.15 Show Point Clouds and Poses

Ports [D]: in Point Clouds Shown in White, optional Point Clouds Shown in Color, optional Poses; no data output. Parameters: Show Normals, Display Interval of Normals (default 20), Z Value Visualization Settings with Upper/Lower Bound.

Visualisation convention: X red, Y green, Z blue, with an overlay such as Pose List 1: size: 8 [D]. Turning on Z-value visualisation with bounds around the top layer makes multi-layer tray unloading legible — you can see the current layer separate from the one below [E].

### 8.16 Reading step status, warnings and timing

| Signal | Meaning |
| --- | --- |
| Step frame colour | Regular / Selected / Warning (yellow frame) / Error [D] |
| Time: <n>ms in the lower-left of a step block | That step's execution time [D] |
| Log panel tabs | Vision (project debug info and errors), Console (control info, including the final TCP and status codes output by the vision system), Operation (user actions) [D] |
| Log levels | D debug, i information, W warning, E error; toggle with the icons at lower-left (blue = shown) [D] |
| Log retention | Default 7 days, configurable at Settings → Options → Common → Log settings, max 99 days [D] |

The yellow "input is empty / no output / auto-fill output" warnings. These exact strings are not documented [?]. The reference project shows several of them, which is normal for a project inspected while idle. Practical reading [E]: they are warnings, not errors — the step ran, found one of its input lists empty (the "nth input" names the port, 1-based), produced nothing meaningful, and emitted a padded/empty output so downstream steps still receive a well-formed list instead of failing. They almost always mean an upstream step returned zero items — recognition found nothing, or a validate+filter chain culled everything. Look upstream, not at the warning step.

Documented mechanisms that behave similarly and are worth knowing [D]:

| Execution flag | Behaviour |
| --- | --- |
| Continue Given No Output | The project continues even if a step produces no output |
| Trigger Control Flow Given No Output / Given Output | Whether the control flow fires when the step has no / has output |
| Notify Procedure Finished When No Output | Whether to notify of empty project output and end the run |
| Reuse Input | Aligns list lengths between input ports by duplicating the shorter one |
| Visualize Outputs | Interlocked with the eye icon on the step block |

A genuine hard failure looks different: an unconnected necessary input port raises a "Procedure Execution Error" pop-up, and the fix is to connect the port [D].

Run vs Continuous Run [D]: Run (Ctrl+R) executes the project once. Continuous Run repeats it per Max number of continuous executions (−1 = indefinitely) and Interval after project execution, set via the gear beside the button. In production the project is triggered by command 101, not by either button. Use Continuous Run for soak testing: does recognition hold as the tray empties, is cycle time stable, does the camera drop frames over 500 cycles [E].

Debug Output [D]: the panel sits in the upper right. Enable the toolbar toggle and click the eye icon on a step (closed → open) to visualise it. Click a step's Single Step Execution button to run that step alone; double-click a data-flow line to display that stream's contents — the fastest way to inspect an intermediate port. Pop out individual windows to watch several steps at once; the chosen 3D viewpoint is retained on the next run. Documented debugging method: when the result is wrong, "locate the Step(s) of error by running the Steps one by one", then tune that step.

## 9. Fixture-side and cell-state logic

This is the part of machine tending that has no equivalent in bin picking, and the part most often bolted on late.

### 9.1 What state the cell has

| State | How it is known | Owner |
| --- | --- | --- |
| Machine cycle complete | M-code / DO from CNC | PLC/robot |
| Door open / closed | Interlock switch | PLC/robot |
| Chuck / vise open / clamped | Pressure or proximity switch | PLC/robot |
| Nest occupied by a finished part | Sensor if fitted; vision ROI check if not (§9.2) | Robot, informed by vision |
| Blank available in the infeed | Vision result count | Vision → robot |
| Correct part number loaded | Label from recognition, or recipe ID | Vision → robot |
| Part seated correctly | Seat-check air switch (best), or post-load vision check | PLC/robot |

Rule [E]: prefer a hard sensor for anything safety- or damage-critical (seat check, chuck clamped). Use vision for what sensors cannot see — which part, where it is, and whether the nest is clear when no sensor exists.

### 9.2 Nest occupancy check with vision

Pattern [E], built entirely from documented steps:

Capture (from a pose that sees the fixture)
   └─ 3D Target Object Recognition            (the finished-part target object)
         └─ Validate Existence of Poses in 3D ROI      ROI = the nest volume
               ├─ Validation Results ─► Trigger Control by Flag  (TriggerWhenTrue)
               │                          └─► UNLOAD branch: Output the part pose
               └─ (no poses in ROI) ──► Trigger Control by Flag  (TriggerWhenFalse)
                                          └─► LOAD branch: nest is clear

Notes:

- Trigger Control by Flag needs a single-element Boolean list [D] — reduce the per-part Bool[] to a count comparison first.

- With an eye-to-hand camera that cannot see inside the machine, run this check on the staging position instead, or fit a sensor. Do not fake it.

- The robot can also learn "nothing to unload" from the protocol: status 1002 (No vision result) or 1003 (No point cloud in ROI) from command 102 [D]. That is often simpler than a branch (§14.3).

### 9.3 Load / unload sequencing with one gripper

1. Vision: is the nest occupied?          → yes
2. Unload: pick finished part from nest, place in outfeed
3. Vision: find next blank (101 → 102/105)
4. Load: place blank into nest, clamp, verify seat
5. Start machine cycle

With a double gripper (archetype A3), steps 2–4 collapse into one trip and the ordering constraint moves into the motion plan — which is the point at which Mech-Viz earns its licence (§10.1).

### 9.4 Error proofing before the door closes

Cheap checks that prevent expensive crashes [E]:

| Check | Method |
| --- | --- |
| Wrong part number picked | Compare the recognition label against the active recipe; the robot receives the label as element 7 of each vision point [D] |
| Part missing from the gripper | Gripper part-present sensor (not vision) |
| Part not seated | Seat-check air pressure switch |
| Chips in the nest | Air blast + a Validate Existence of Poses in 3D ROI check, or a point-cloud count threshold |
| Short / oversized part | Enable Point clouds of matched target objects and compare dimensions [D] |
| Machine still holding a part | §9.2 |

### 9.5 Bin-empty and end-of-tray

Three documented ways to tell the robot "nothing left" [D]:

- Status codes from 102: 1002 No vision result, 1003 No point cloud in ROI.

- Vision-Move-specific: 1044 / 2039 No vision point for the Vision Move Step.

- The Notify step + command 601 — a positive-integer message pushed from the project (§12.7).

Prefer 1 [E]: it needs no extra project structure and it is unambiguous to the robot program. Use Notify when you need to distinguish several application states ("tray empty" vs "layer complete, index the tray") in one cycle.

## 10. Motion planning: Path Planning step vs Mech-Viz

### 10.1 The decision

| Use the Mech-Vision Path Planning step when | Use a full Mech-Viz project when |
| --- | --- |
| The robot program owns the whole cycle | You need branching by part number or fixture state (Branch by Message + command 203) |
| The vision-dependent motion is a short approach–pick–depart near one bin | Placement into a chuck/fixture with orientation constraints and symmetry-aware grasp selection |
| You want one fewer piece of software to launch and keep alive on the IPC | Counters / DO sequencing owned by the vision side, or gripper DO lists returned to the robot (206) |
| Standard Interface or Adapter communication [D] | Multi-station or dual-gripper cycles |
| — | Master-Control — the Path Planning step cannot do master control |

Supporting evidence: the step's documented scope is planning "near the vision point" / "near the workobject" [D]; a Mech-Mind community answer (semi-official, [?]) states the Path Planning step is "limited to simple applications" and non-master-control only, while Mech-Viz enables "more complex projects (with multiple branches, oriented placing, interaction with adapter etc.)".

Default for machine tending [E]: start with the Path Planning step (archetypes A1, A2, A6). Move to Mech-Viz when you hit A3/A4/A5 complexity. The reference Drill_Tap project uses the Path Planning step, which is the right call for a single-station load with a fixed cycle owned by the robot.

### 10.2 Mech-Viz project structure

A project holds project resources — robots, tools, workobjects, scene objects — plus the workflow (the visual robot-motion program). Reference frames: World (fixed at the simulation centre), Robot Base, Robot TCP, Scene Object [D]. A solution is the top-level container bundling Mech-Vision project(s), one Mech-Viz project, robot configuration and calibration files [D].

Steps are function modules connected exit-port → entry-port. Categories: General Parameters of Steps, Basic Move, DI DO, Logical Topology, Pallet, Robot Utilities, Service, Utilities, Trajectory, Vision, Others [D].

Canonical pick-and-place workflow order [D]:

Home (move) → Waypoint (move) → visual_look_1 → relative_move_1 (approach, e.g. Z −200 mm)
→ visual_move_1 → Set_DO_Pick (set_do) → wait_1 → relative_move_2 (depart)
→ relative_move_3 (approach place) → move_1 (place) → Set_DO_Place → wait_2
→ relative_move_4 (depart) → Return_Home

Relative Move takes its reference from "Next" (upcoming target) or "Current" (previous target) [D]. Internal names are visual_look / visual_move; the UI labels are Vision Look / Vision Move [D].

For machine tending, extend this skeleton with a branch after visual_look for load vs unload, and put the machine handshake outside Mech-Viz wherever possible (§2.3) [E].

Parameter-level references for the Checker, Counter and Message steps were not reachable for this handbook [?]; Branch by Message is confirmed indirectly through command 203 [D].

### 10.3 Tool, TCP and workobject configuration

Tool [D]: add the tool model (usually STL) under Collision Models, rotating axes if the import orientation is wrong, then create an entry under End Effectors with a name and that model. TCP defaults to the flange centre — "increase the value of Z until the TCP is at the tip of the tool" — but in real deployments "please input the calibrated TCP values", or use Update TCP from Robot to pull the controller's active TCP.

Workobject parameters [D]: Rotation Symmetry (how the object coincides with itself under rotation — this drives which pick orientations are legal), Picking Relaxation (whether exact positioning is required at grasp), Optimal Picking Solution Strategy (how far the tool may rotate). Hard constraint: the workobject's Z-axis must oppose the TCP's Z-axis during grasping; symmetric objects "can be grasped at multiple orientations".

Machine-tending note [E]: for turned studs, shafts and drill blanks — bodies of revolution — setting rotational symmetry correctly is what unlocks reachable grasps on a chuck-loading approach. Get it wrong and the planner rejects perfectly good picks as unreachable because it insists on one arbitrary clock position.

Scene objects to model [D]: safety fence, picking bin, tray, camera, camera stand. For machine tending add the machine body, the open door envelope, the chuck/vise, and any tailstock or tool turret that can be inside the swept volume [E].

### 10.4 Collision detection

Pairs checked by default and not configurable [D]: robot links ↔ robot tool, and scene objects ↔ robot tool.

Tool model format matters [D]:

| Format | Geometry | Detection | Thresholds |
| --- | --- | --- | --- |
| STL | Mesh (linked triangles) | Surface collisions only | Collision points threshold (default 0), Collision area threshold (default 0) |
| OBJ | Assembly of convex polyhedra (solid) | "Precise and efficient"; detects contact with the tool interior | Collision volume threshold (default 5 mm³) |

Caution [D]: "OBJ models that contain concave polyhedra cannot be used for collision detection anymore." For a two-jaw chuck-loading gripper, decompose it into convex pieces or accept STL surface-only detection.

Both formats expose a pose preference: minimum X/Y-axis rotation vs minimum collision within the X/Y rotation range [D]. Threshold tuning is an explicit trade: higher thresholds raise pick success but risk disturbing neighbouring parts; lower thresholds guarantee clean paths at a throughput cost [D].

Point cloud collision detection [D]: the cloud is converted to an octree of cubes centred on points; the key parameter is "Resolution of octree converted from point cloud" (cube edge length; the docs illustrate 10 mm vs 2 mm). Shorter edge = more accurate, slower. Prerequisite: the Mech-Vision project must include the "Send Point Cloud to External Service" step. Default values and the separate volume/point thresholds for the cloud are not documented [?].

Machine-tending practice [E]:

- Model the gripper with the part in it for the loaded moves, or enable Remove Point Cloud of Irregular Shape in the Path Planning step so the held part is checked (§8.13).

- Model the machine door aperture generously. The most common real collision in machine tending is the part, not the gripper, catching the door frame or the chuck jaws on the way in.

- Simulate at production speed and with a full bin and a nearly empty one; collision-free at 30 % speed with three parts left proves nothing.

## 11. The Output step as an interface contract

Treat Output as an interface specification signed by the vision engineer and the robot programmer [E].

### 11.1 Choose the type by who plans motion

2.1.x Port Type [D]:

| Value | Use |
| --- | --- |
| Predefined (vision result) — default | Mech-Vision does vision only; result goes to Mech-Viz or an external service. Suits Master-Control, Standard Interface or Adapter |
| Predefined (robot path) | Mech-Vision does vision and path planning. Only the Path Planning step can feed it, and receiving these results via Mech-Viz is not supported |
| Custom | Customise the data sent; prefer predefined keys where possible |

⚠ 2.2.x renames it Output Type with four values: Vision results for picking (default), Vision results for trajectory, Robot path for picking, Custom [D].

### 11.2 Custom port keys (2.1.x)

All array format [D]:

| Key | Meaning | Required |
| --- | --- | --- |
| poses | Object poses | Yes |
| labels | Object labels, same length as poses | No |
| sizes | 3D dimensions, same length as poses or length 0 | No |
| offsets | Pick-point offset relative to object centre point | No |
| objectIndexes | Object index | No |
| scene_object_names / _sizes / _poses | Scene objects to update | No |
| workobject_data | Target object picking strategy for Mech-Viz; connect to the Picking Strategy output of Generate Picking Strategy | Yes when using Mech-Viz |

Interaction rules [D]:

- workobject_data alone → only Mech-Viz can parse it; Standard Interface commands 100, 102, 105, 110 get nothing.

- Other ports alone → Mech-Viz cannot parse and errors with "No pick points have been obtained".

- Both → Mech-Viz parses only workobject_data, while the other data still reaches the Standard Interface.

### 11.3 Other parameters that matter in production

[D] unless noted:

| Parameter | Production setting |
| --- | --- |
| Send Point Cloud to External Service | Default on; documented as "usually used for debugging or checking the running result". Turn it off after commissioning to save bandwidth every cycle — unless Mech-Viz point-cloud collision detection needs it (§10.4), in which case it must stay on |
| Update Scene Object | On when the bin/pallet pose is re-measured, so the same pose informs collision checking and the robot |
| Point Cloud in Camera Frame | Default on |
| Point Cloud Type | CloudXYZRGB default |
| Remove Point Cloud of Irregular Shape + Search Radius (3 mm) | As §8.13 |
| Other Inputs | Adds Pick Point Labels, Target Object Dimensions, Pick Point Offsets |
| Auto-Correct Accuracy Drift in Vision System | Default on; only shown after drift correction is deployed in the Error Analysis Tool. Keep it on for ±1 mm class cells |

### 11.4 What the robot actually receives

Critical conversions performed on the way out [D]. Note the actor: items 1 and 2 are performed by the vision system when it converts the Output step's poses port data into robot TCP for Standard Interface commands 100/102 — they are not done by the Output step itself, and poses handed to Mech-Viz remain quaternions.

- Object poses are converted from quaternions to Euler angles.

- The pose is rotated 180° about X to orient its Z-axis downward.

- "If the first input port of the Output Step is Object Center Points, the Output Step will convert the object center points into the corresponding pick points" — so the poses the robot receives are pick points, not centre points.

- Label must be an integer-formatted string; if no label is available it defaults to 0. A non-integer label raises status 1017.

- Tool ID defaults to 0; "in most cases, vision points output by Mech-Vision do not have information of the object tool ID". For waypoints from 105/205 the tool ID comes from the path planning tool / Mech-Viz.

## Write these five facts into the interface document you hand the robot programmer [E]. Items 2 and 3 in particular explain most "the pose looks 180° out" and "it picks 40 mm above the part" reports.

## 12. Robot ↔ vision communication (Standard Interface)

### 12.1 Ground rules

[D] unless noted:

- The robot / PLC / host is the client; the vision system is the server. The client always initiates; the vision system only replies and never pushes unsolicited data (the one exception is the Notify mechanism, §12.7).

- Formats: ASCII (comma-delimited, \r terminator, e.g. 103,1,2\r) or HEX (fixed 64 bytes, big- or little-endian; short data zero-padded, extra fields ignored).

- Units: joint positions in degrees; flange/tool pose [x, y, z, a, b, c] with x/y/z in mm and a/b/c as Euler angles in degrees in the selected brand's convention.

- Batching: default max 20 poses per reply, configurable in Robot Communication Configuration → Next → Advanced Settings, hard limit 30. Default reply timeout 10 s.

- Robot-side buffer: the robot can hold up to 100 vision points or waypoints from the vision system.

- Protocol format is brand-pinned: FANUC V7–V9 = HEX big-endian; FANUC V10 = HEX little-endian; ABB (RobotWare 6) = HEX little-endian; KUKA = HEX little-endian; YASKAWA = ASCII.

- The interface service must be running — after Apply, verify the Robot Communication Configuration toggle on the toolbar is flipped and blue, or nothing responds.

### 12.2 Configuration checklist

In Mech-Vision → Robot and Communication → Robot Communication Configuration [D]:

- Select robot → Listed robot → robot model → Next.

- Interface service type = Standard Interface.

- Protocol = TCP Server.

- Protocol format per §12.1.

- Port number — free, and see §3.4 for the FANUC V10 ≤ 32767 constraint.

- Robot integration → Open program folder — the source of the MM_* files to load onto the controller.

- Optionally Auto enable interface service when opening the solution (recommended for production).

- Apply, then confirm the toolbar toggle.

For TCP Server the IP address field is 0.0.0.0 and the robot program needs the IPC's IP plus this port [D].

Advanced Settings holds maximum number of poses to obtain each time (default 20, limit 30), Timeout for getting Mech-Vision data (default 10 s), and Property Configuration (the property_config file for commands 207/208) [D].

### 12.3 Command set — Mech-Vision

All [D], 2.2.1 names. Legacy 1.7.x names in Appendix B.

| # | Name | Sent | Reply | Success |
| --- | --- | --- | --- | --- |
| 100 | Trigger Mech-Vision Project and Get Results | 100, project ID, recipe ID, returned data format, joint positions, flange pose | Depends on format 1–4 | 1100 |
| 101 | Trigger Mech-Vision Project | 101, project ID, expected number of vision points or waypoints, robot pose type, robot pose | 101, status | 1102 |
| 102 | Get Vision Results | 102, project ID | 102, status, transmit status, count, reserved, vision point 1 (TCP, label, tool ID), … | 1100 |
| 103 | Switch Mech-Vision Parameter Recipe | 103, project ID, recipe ID (1–99) | 103, status | 1107 |
| 104 | Switch Mech-Vision Solution | 104, solution ID | 104, status | 1104 [?] |
| 105 | Get Planned Path from Mech-Vision | 105, project ID, waypoint pose type (1 = JPs, 2 = tool pose) | 105, status, transmit status, count, position of "Vision Move" in path, waypoint 1 … | 1103 |
| 106 | Get Gripper DO List from Mech-Vision | 106, project ID, number of vacuum gripper sections | 106, status, DO 1 … DO 64 | 1106 |
| 110 | Get Custom Data from Mech-Vision | 110, project ID | 110, status, transmit status, N, pose, label, element 1 … N | 1100 |
| 111 | Get Vision Move Data from Mech-Vision | 111, project ID, waypoint pose type | 111, status, transmit status, waypoint type, pose, motion type, tool ID, Vision Move data | 1103 |
| 501 | Input Object Dimensions | 501, project ID, length, width, height (mm) | 501, status | 1108 |
| 503 | Input Poses to Project | 503, project ID, Step name, pose | 503, status | 1110 |
| 504/505 | Set / Get Numeric Global Variable | 504, ID, value / 505, ID | status (+ value) | 1111 [?] |
| 506/507 | Set / Get String Global Variable | as above | as above | 1111 [?] |
| 601 | Get Message from Notify Step | 601 | 601, message | (no status code) |
| 701 | Calibration | 701, calibration status, flange pose, joint positions | 701, status, calibration status, next flange pose, next joint positions | 7101 |
| 901 | Get System Status | 901 | 901, status, Mech-Vision status, Mech-Viz status, camera status | 9100 |

Command 101 robot pose type [D]:

| Type | Pose sent | Scenario |
| --- | --- | --- |
| 0 | 0,0,0,0,0,0 — Path Planning start point = the tool's Home point | Eye-to-hand, no pre-capture needed |
| 1 | Current JPs and flange pose | Eye-in-hand; recommended for most scenarios except gantry |
| 2 | Current flange pose | Recommended for gantry robots |
| 3 | User-defined joint positions, used as the Path Planning start point | Eye-to-hand where images are captured beforehand |

Command 901 decode [D]: Mech-Vision 0 = not opened / 1 = opened; Mech-Viz likewise; camera 0 = at least one camera disconnected / 1 = all connected. Poll it at cell start-up before the first cycle [E].

### 12.4 Command set — Mech-Viz

All [D]:

| # | Name | Sent | Success |
| --- | --- | --- | --- |
| 200 | Trigger Mech-Viz Project and Get Planned Path | 200, Branch-by-Msg Step ID, exit port, waypoint pose type, joint positions, flange pose | 2100 |
| 201 | Trigger Mech-Viz Project | 201, robot pose type, robot pose | 2103 |
| 202 | Stop Mech-Viz Project | 202 | 2104 |
| 203 | Set Exit Port for Branch by Msg | 203, Step ID, exit port | 2105 |
| 204 | Set Current Index | 204, Step ID, Current Index | 2106 |
| 205 | Get Planned Path from Mech-Viz | 205, waypoint pose type | 2100 |
| 206 | Get Gripper DO List from Mech-Viz | 206, number of vacuum gripper sections | 2102 |
| 207 / 208 | Read / Set Mech-Viz Step Parameter | 207, Config ID / 208, Config ID | 2109 / 2108 |
| 210 | Get Vision Move Data or Custom Data from Mech-Viz | 210, returned data format (1–4) | 2100 |

Command 201 robot pose type [D]: 0 = [0,…,0], the simulated robot moves from its set home (eye-to-hand); 1 = current JPs + flange pose (recommended eye-in-hand); 2 = JPs customised by the robot, i.e. a taught image-capture point — "robot pose type should be set to 2 for projects in eye to hand mode", and it allows planning ahead, shortening cycle time.

Two off-by-one traps, both verbatim from the docs [D]:

- 203 / 200 exit port: "When the parameter value is set to 'N', the Mech-Viz project exits from the port with an ID of 'N-1'." So 203, 2, 1 selects port 0 of Step 2.

- 204 Current Index: "When this parameter value is set to 'N', the Current Index of the corresponding Step is 'N-1'."

207/208 Config IDs come from a property_config file [D]:

read,  <Config ID>, <Step ID>, <parameter key name>
write, <Config ID>, <Step ID>, <parameter key name>, <parameter value>

Examples: read, 5, 3, xCount, write, 6, 3, xOffset, 0.000000. Several write lines may share a Config ID (setting multiple parameters in one command); read IDs must be unique. Key names come from Mech-Viz Tools → Key Query Tool (Developer mode). The interface service must be restarted after editing property_config [D].

### 12.5 Calling sequence — get this wrong and nothing works

[D], verbatim constraints:

Mech-Vision chain: 103 and/or 501 (before) → 101 → one of 102 / 105 / 110. "Command 102, Command 105, and Command 110 cannot be used at the same time."

Mech-Viz chain: 207/208 (before) → 201 → 204 then 203 (204 before 203) → 205 or 210 → after the project stops, 206. "Command 205 and Command 210 cannot be used at the same time." For index-type steps the prescribed order is 201 → 204 → 203 "to ensure that Mech-Viz has enough time to set the Current Index value."

Other ordering constraints [D]: 104 before 100/101; 106 after 100/105/111; 601 "immediately AFTER Command 101 or Command 201"; 503 before 101.

PLC handshake (PROFINET / EtherNet/IP / Snap7 / MC / Modbus), verbatim [D]: write command number + parameters to registers → PLC sets Command_Trigger = 1 → vision reads → vision sets Trigger_Acknowledge = 1 → PLC sets Command_Trigger = 0 → vision sets Trigger_Acknowledge = 0.

### 12.6 Reading a vision result

102, status, transmit status, count, reserved, vision point 1 … [D]:

- transmit status — 0 = not all vision points obtained; 1 = all obtained. "If the value is 0, repeat sending the command until the value becomes 1. If not all vision points are obtained when Command 101 is called, the remaining vision points will be cleared."

- count — the number in this reply (≤ 20 by default, 30 max).

- reserved — not in use, value 0.

- vision point — exactly 8 elements: 1–6 = TCP, 7 = label, 8 = tool ID.

Documented 22-point exchange [D]:

→ 101, 1, 0, 1, -0, -20.6323, -107.8121, -0, -92.8181, 0.0016
← 101, 1102
→ 102, 1     ← 102, 1100, 0, 20, 0, 95.7806, 644.5677, 401.1013, 31.1206, …
→ 102, 1     ← 102, 1100, 1,  2, 0, 315.2017, 592.1261, 399.6052, 126.1960, …

Custom data is not available through 102 — use 110, which returns one vision point per call; call it repeatedly for several points. N = the sum of the column counts of all custom ports, arranged in alphabetical order of port names, and the Output step's Port Type must be Custom with the poses port present [D].

For 105/205, note the documented semantic shift of position of "Vision Move" in planned path across multiple replies: in the first response it is the position within the entire path; in later responses it is the position among the remaining waypoints [D]. Also set 101's expected count to 0 before using 105, or you must call 105 once per waypoint [D].

motion type: 1 = joint (MOVEJ), 2 = linear (MOVEL). tool ID = -1 means no tool used at that waypoint [D].

Terminology correction [D]: there is no parameter called sendPose. The robot-side parameter is SendPos_Type (YASKAWA sendpos_type) and it selects which robot pose is sent to the vision system (0–3). Batching is governed by Pos_Num_Need, the Advanced Settings pose cap, and the transmit-status loop.

### 12.7 Notify — the only vision→robot push

There is no "empty tray" command [D]. To let the project tell the robot "condition X occurred", use the Notify step with command 601:

- Mech-Vision side [D]: connect Notify to the right of another step (e.g. Output); in the Output step's parameters select Trigger Control Flow Given Output; in the Notify step set Service Name = Standard Interface Notify (a required literal) and Message = a positive integer (e.g. 1001).

- Mech-Viz side [D]: place Notify in the workflow, select Standard Interface as the receiver, set Message to a positive integer.

- Reply is 601, <message> — no status code [D].

- Timing hazard, verbatim [D]: "When the Notify Step is executed in the Mech-Vision or Mech-Viz project, the message remains in the buffer of the vision system for only three seconds." Hence the requirement to call 601 immediately after 101 or 201.

- Range 6001–6199 is documented as "status codes that can be customized in Mech-Vision" — the sanctioned block for application states such as "tray empty" [D]; how to author them is not documented [?].

Practice [E]: the 3-second buffer makes Notify fragile in a cell where the robot may be mid-move. Prefer status codes (§9.5) for anything the robot must not miss, and reserve Notify for advisory messages such as "layer complete, index the tray".

## 13. Robot-side programs

### 13.1 The universal shape

All four brands follow the same pattern [D]:

- A background/driver module (socket + protocol) plus thin wrapper routines named MM_* called from the user's motion program.

- Results land in registers / variables, never as return values.

- Two-stage retrieval is mandatory: a Get command fetches data into robot memory, then a Store command copies one indexed item into a position register or variable. FANUC states it explicitly: "The returned vision result is saved to the robot memory and cannot be directly obtained. To access the vision result, you must store the vision result in a subsequent step."

- Programs live on the IPC at Communication Component/Robot_Interface/<BRAND>/, reachable via Robot Communication Configuration → Robot integration → Open program folder; examples in .../<BRAND>/sample/ [D].

### 13.2 Generic machine-tending cycle (brand-neutral pseudocode)

# ---- once, at controller start / program init ----
MM_INIT_SOCKET(ipc_ip, port, timeout)
MM_GET_STATUS(status)                     # cmd 901
IF status != 9100  -> alarm "vision system not ready"
IF vision_status != 1 OR camera_status != 1 -> alarm

# ---- per part ----
MOVE J  home
WAIT    DI[machine_cycle_done] AND DI[door_open]

# --- unload the finished part (taught position or vision) ---
GOSUB   unload_finished_part

# --- select the recipe for the active part number (optional) ---
MM_SWITCH_MODEL(project=1, recipe_id=active_recipe, status)   # cmd 103
IF status != 1107 -> alarm

# --- trigger vision from the capture pose ---
MOVE L  capture_pose                       # eye-to-hand: not required; see pose type 0/3
MM_START_VIS(project=1, pos_num_need=0, sendpos_type=2, pr=..., status)   # cmd 101
IF status != 1102 -> GOTO vision_error

# --- fetch either poses or a planned path, never both ---
IF using_path_planning_step:
    MM_GET_VISPATH(project=1, jps_pos=1, pos_num, vis_pos_num, status)   # cmd 105
    IF status != 1103 -> GOTO vision_error
    FOR i = 1 TO pos_num
        MM_GET_JPS(i, PR[60], R[label], R[toolid])       # store one waypoint
        IF i == vis_pos_index THEN  close_gripper  ENDIF
        MOVE (motion_type_i)  PR[60]
    ENDFOR
ELSE:
    MM_GET_VIS(project=1, pos_num, status)               # cmd 102
    IF status != 1100 -> GOTO vision_error
    MM_GET_POS(1, PR[60], R[label], R[toolid])           # store vision point 1
    IF R[label] != expected_label_for_recipe -> GOTO wrong_part
    MOVE J  approach_via_offset(PR[60], z=+100)
    MOVE L  PR[60]      slow
    close_gripper
    MOVE L  approach_via_offset(PR[60], z=+100)
ENDIF

# --- load the machine ---
MOVE J  door_entry
MOVE L  nest_approach
MOVE L  nest_seat        slow
open_gripper / release
WAIT    DI[seat_check_ok]                 # hard sensor, not vision
SET     DO[chuck_close]
WAIT    DI[chuck_clamped]
MOVE L  nest_retreat
MOVE J  clear_of_door
SET     DO[cycle_start]
JUMP    top_of_loop

vision_error:
    # decode the status code and branch — see §14
wrong_part:
    # place the part back / to a reject chute, then retry

Notes [E]: keep MM_INIT_SOCKET out of the cycle loop — it belongs in initialisation. Keep the IF status != … check after every MM_* call; the example programs do this, and it is the difference between a cell that stops safely and a cell that moves to a stale pose.

### 13.3 FANUC

Prerequisites [D]: controller system software V7.5, 7.7, 8.x, 9.x or 10.1. R651 or R632 (KAREL) must be installed. R648 (User Socket Msg) must be installed. Ethernet to motherboard CD38A (Port 1) or CD38B (Port 2).

Loading [D]: copy the contents of the FANUC folder (not the folder) to the root of a ≤ 32 GB FAT32 flash drive; on the TP: MENU → FILE → F5 UTIL → Set Device → USB Disk (UD1:) or USB on TP (UT1:), select INSTALL, ENTER, F4 YES. Success message: "Programs Loaded". Test with MM_COMTEST (args: robot port 1–8, IPC IP, IPC port, timeout in minutes). Changing the robot port number for the first time requires a controller restart. The individual installed file names are not published [?].

Routines (exact signatures) [D]:

| Feature | Routine |
| --- | --- |
| Init communication | MM_INIT_SKT(C_Tag, Ip_Addr, Svr_Port, Time_Out) — C_Tag is a string '1'–'8'; Time_Out in minutes; V10 Svr_Port 1–32767 |
| Run Mech-Vision project | MM_START_VIS(Job, Pos_Num_Need, SendPos_Type, Pr_Num, MM_Status) |
| Get vision result | MM_GET_VIS(Job, Reg_Pos_Num, MM_Status) |
| Store result/path (TCP) | MM_GET_POS(Serial, Pr_Num, Reg_Label, Reg_ToolId) — Serial is 1-based |
| Store path (joint positions) | MM_GET_JPS(Serial, Pr_Num, Reg_Label, Reg_ToolId) |
| Switch parameter recipe | MM_SET_MOD(Job, Model_Num, MM_Status) |
| Get planned path (Mech-Vision) | MM_GET_VISP(Job, Jps_Pos, Reg_Pos_Num, Reg_VPos_Num, MM_Status) |
| Get / store custom data | MM_GET_DY_DT(Job, Reg_Pos_Num, MM_Status) / MM_GET_DYPOS(Serial, Pr_Num, Reg_Label, Reg_UserData) |
| Get / set gripper DO list | MM_GET_DL(Resource, Block_Num, MM_Status) / MM_SET_DL(Loop_Index) |
| Run / stop Mech-Viz | MM_START_VIZ(SendPos_Type, Pr_Num, MM_Status) / MM_Stop_Viz(MM_Status) |
| Set branch / index | MM_SET_BCH(Branch_Num, Export_Num, MM_Status) / MM_SET_IDX(Skill_Num, Index_Num, MM_Status) |
| Get planned path (Mech-Viz) | MM_GET_VIZ(Jps_Pos, Reg_Pos_Num, Reg_VPos_Num, MM_Status) |
| Vision Move / plan data | MM_GET_PLNDT(...) / MM_GET_PLJOP(Serial, Jps_Pos, Pr_Num, Reg_MoveType, Reg_ToolNum, Reg_Speed, Reg_UserData, Reg_PlanRes) |
| Read / set Viz step parameter | MM_GET_PROP(Read_id, MM_Status, Reg_Viz_Prop) / MM_SET_PROP(Write_id, MM_Status) |
| Input object dimensions / poses | MM_SET_BS(Job, Length, Width, Height, MM_Status) / MM_SET_VISP(Job, Step_Name, Pr_Num, MM_Status) |
| Notify / calibration / status | MM_GET_NTFY(MM_NotifyMsg) / MM_CALIB(...) / MM_GET_STAT(MM_Status) |

Canonical TP flow, from the official MM_S1_Vis_Basic [D]:

UFRAME_NUM=0 ; UTOOL_NUM=1 ;
J P[1] 100% FINE ;                            ! home
CALL MM_INIT_SKT('8','127.0.0.1',30000,5) ;   ! once only
L P[2] 1000mm/sec FINE ;                      ! image-capture pose
CALL MM_START_VIS(1,0,2,10,53) ;
IF (R[53]<>1102),JMP LBL[99] ;
CALL MM_GET_VIS(1,51,53) ;
IF R[53]<>1100,JMP LBL[99] ;
CALL MM_GET_POS(1,60,70,80) ;                 ! PR[60]=TCP, R[70]=label, R[80]=toolId
J P[3] 50% CNT100 ;
L PR[60] 1000mm/sec FINE Tool_Offset,PR[1] ;  ! approach
L PR[60]  300mm/sec FINE ;                    ! pick
...
LBL[99:vision error] ;

FANUC ships 21+ example programs, MM_S1_Vis_Basic … MM_S22_Vis_As_Uframe [D]. The ones worth reading for machine tending: MM_S6_Viz_ErrorHandle (error handling), MM_S9_Viz_RunInAdvance (pipelined capture to cut cycle time), MM_S13_Vis_MoveInAdvance, MM_S15_Viz_GetDoList, MM_S19/S20_PlanAllVision, MM_S8/S10_Viz_Subtask, MM_S21_1Robot_3IPC_Sequentially.

### 13.4 ABB (RAPID)

Prerequisites [D]: IRC4 or IRC5; RobotWare 6.02–6.15; control module 616-1 PC Interface. Ethernet to controller X6 (WAN).

Files loaded to task T_ROB1 [D]: MM_Module.mod (program module), MM_Auto_Calib.mod (calibration), MM_Com_Test.mod (comms test). RobotWare 7: change the extension from .mod to .modx. Auto-load via Communication Component\tool\Robot Program Loader (back up first, then Load the Standard Interface program → Load with one-click), or manually via the TP / RobotStudio.

ABB is the only brand with explicit open/close socket routines [D].

| Feature | Routine |
| --- | --- |
| Init / open / close | MM_Init_Socket IP_Address, Server_Port, Time_Out; (Time_Out in seconds) · MM_Open_Socket; · MM_Close_Socket; |
| Run project / get result | MM_Start_Vis Job, Pos_Num_Need, SendPos_Type, MM_J, MM_Status; · MM_Get_VisData Job, Pose_Num, MM_Status; |
| Store pose / JPs | MM_Get_Pose Serial, MM_P, MM_Label, MM_ToolId; · MM_Get_Jps Serial, MM_J, MM_Label, MM_ToolId; |
| Recipe / planned path | MM_Switch_Model Job, Model_Number, MM_Status; · MM_Get_VisPath Job, Jps_Pos, Pos_Num, VisPos_Num, MM_Status; |
| One-shot trigger + fetch | MM_Lite_Vis Job, Model_Number, Recv_Data_Type, MM_Status, \MM_J, \Pos_Num, \VisPos_Num; — max 20 poses |
| Custom data | MM_Get_DyData job, Pos_Num, MM_Status; · MM_Get_DyPose Serial, MM_P, MM_Label; |
| DO list | MM_Get_DoList Resource, BlockNum, MM_Status; · MM_Set_DoList Loop_Index, Serial, Go16; |
| Mech-Viz | MM_Start_Viz, MM_Stop_Viz, MM_Set_Branch, MM_Set_Index, MM_Get_VizData, MM_Lite_Viz, MM_get_plandata, MM_Get_PlanPose |
| Properties / dimensions | MM_Get_Property, MM_Set_Property, MM_Set_BoxSize |
| Notify / calib / status | MM_Get_Notify Msg, MM_Status; · MM_Calib Move_Type, Pos_Jps, Wait_time \Ext; · MM_Get_Status MM_Status; |

Recv_Data_Type for the Lite routines: 1 = vision points without custom data (then MM_Get_Pose); 2 = with custom data (then MM_Get_DyData); 3 = waypoints as joint positions (then MM_Get_Jps); 4 = waypoints as TCP (then MM_Get_Pose) [D].

Canonical RAPID core [D]:

MoveAbsJ home\NoEOffs, v3000, fine, gripper1;
MM_Init_Socket "127.0.0.1", 50000, 300;      ! once only
MoveL camera_capture, v1000, fine, gripper1;
MM_Open_Socket status;
IF status = 3099 THEN TPWrite "MM: Communication Error"; STOP; ENDIF
MM_Start_Vis 1, 0, 2, snap_jps, status;
IF status <> 1102 THEN TPWrite "MM: Status Error"; STOP; ENDIF
MM_Get_VisData 1, pose_num, status;
IF status <> 1100 THEN Stop; ENDIF            ! 1003 no cloud in ROI / 1002 no result
MM_Close_Socket;
MM_Get_Pose 1, pickpoint, label, toolid;
MoveJ pick_waypoint, v1000, z50, gripper1;
MoveL RelTool(pickpoint, 0, 0, -100), v1000, fine, gripper1;
MoveL pickpoint, v300, fine, gripper1;

Note the pattern: open socket → commands → close socket → then store poses. The docs warn that leaving the socket open for a long time can produce an "abnormally closed" error [D].

### 13.5 KUKA (KRL)

Prerequisites [D]: 6-axis; KR C4 or KR C5; KSS 8.2 / 8.3 / 8.5 / 8.6 / 8.7; add-on Ethernet KRL, version pinned to KSS:

| KSS | Ethernet KRL |
| --- | --- |
| 8.2 / 8.3 | 2.2.8 |
| 8.5 | 3.0.3 |
| 8.6 | 3.1.2.29 |
| 8.7 | 3.2.2.16 |

Ethernet port: X66 (KR C4 Compact), KLI (other KR C4), XF5 (KR C5). Expert mode required (default password kuka) [D].

Files and destinations [D]: mm_module.src + .dat → KRC:\R1\mm (create mm if absent); MM_COMTEST.src + .dat → same; XML_Kuka_MMIND.xml → C:\KRC\ROBOTER\Config\User\Common\EthernetKRL; sample/ for examples.

<ETHERNETKRL>
  <CONFIGURATION>
    <EXTERNAL><IP>192.168.1.121</IP><PORT>50000</PORT></EXTERNAL>
    <INTERNAL><ALIVE Set_Flag="873"/></INTERNAL>
  </CONFIGURATION>
  <RECEIVE><RAW><ELEMENT Tag="MMRecv" Type="BYTE" Set_Flag="871" Size="660" /></RAW></RECEIVE>
  <SEND><RAW><ELEMENT Tag="MMSend" Type="BYTE" Size="660"/></RAW></SEND>
</ETHERNETKRL>

<IP> = the IPC, <PORT> = the configured host port; flag 873 = connected (ALIVE), flag 871 = data received; payload buffer 660 bytes each way [D].

Routines [D]: MM_Init_Socket(XML_Name, Alive_Flag, Recv_Flag, Time_Out) — e.g. MM_Init_Socket("XML_Kuka_MMIND",873,871,60), Time_Out in seconds, XML_Name case-sensitive without extension. Then MM_Start_Vis, MM_Get_VisData, MM_Get_Pose, MM_Get_Jps, MM_Switch_Model, MM_Get_Vispath, MM_Get_Dy_Data, MM_Get_DyPose, MM_Get_DoList, MM_Set_DoList, MM_Start_Viz, MM_Stop_Viz, MM_Set_Branch, MM_Set_Index, MM_Get_VizData, MM_Get_PlanData, MM_Get_PlanPose, MM_Get_Property, MM_Set_Property, MM_Set_BoxSize, MM_Get_Notify, MM_Calib, MM_Get_Status.

KUKA-only and very useful for machine tending [D]: MM_Get_Wobj(Serial, MM_Frame_num, MM_Label, MM_ToolId) stores the vision pose directly into a base coordinate variable — e.g. MM_Get_Wobj(1,10,label,toolid) writes base variable 10. That lets you express the whole insertion motion as a base-relative sub-program, which is exactly how you want a chuck-loading move written [E].

### 13.6 YASKAWA (INFORM + MotoPlus)

Prerequisites [D]: 6-axis. DX200 → DN3.16.00A-00, YRC1000 → YAS2.94.00-00, YRC1000micro → YBS2.31.00-00. ETHERNET option must be USED and MotoPlus APPLI. AUTOSTART AT POWER ON must be ENABLE. Ethernet: YRC1000 → LAN2 (CN106) on the CPU board (LAN1 is pendant-only; LAN3 if LAN2 is occupied); DX200 → CN104.

Files [D]: mm_module_yrc1000.out / mm_module_dx200.out (MotoPlus background application), JBI/ (the foreground MM_* jobs), sample/. Load the .out in maintenance mode via MotoPlus APL. → DEVICE USB: Pendant → LOAD(USER APPLICATION); verify in FILE LIST. Load jobs via EX. MEMORY FOLDER → JBI → EX. MEMORY LOAD → JOB → EDIT → SELECT ALL. Only one .out may be loaded on most controllers — delete the existing one first or you get FATAL:MM:SOCKET_OPEN_ERROR. Test job MM_COMTEST (edit line 0001 for IP/port; default port 50000).

Arguments are passed as a single semicolon-delimited string via ARGF [D]:

| Feature | Call |
| --- | --- |
| Init / open / close | MM_INIT_SOCKET("IP;Port;Time_Out") (Time_Out in minutes) · MM_OPEN_SOCKET · MM_CLOSE_SOCKET |
| Run project / get result | MM_START_VIS("job;pos_num_need;sendpos_type;prNum;regStatus") · MM_GET_VISDATA("Job;Pose_Num;MM_Status") |
| Store pose / JPs | MM_GET_POSE("Index;Robtarget;Label;Tool_Id") (P variable type must be robot) · MM_GET_JPS("Index;Jointtarget;Label;Tool_Id") (joint/pulse) |
| Recipe / path | MM_SET_MODEL(...) · MM_GET_VISPATH("job;GetPos_Type;Pos_Num;VisPos_Num;regStatus") |
| Custom data / DO list | MM_GET_DYDATA, MM_GET_DYPOSE, MM_GET_DOLIST, MM_SET_DOLIST |
| Mech-Viz | MM_START_VIZ, MM_Stop_Viz, MM_SET_BRANCH, MM_SET_INDEX, MM_GET_VIZDATA, MM_GET_PLANDATA, MM_GET_PLANPOSE |
| Properties / dimensions / notify / calib / status | MM_GET_PROPERTY, MM_SET_PROPERTY, MM_SET_BOXSIZE, MM_GET_NOTIFY, MM_CALIB, MM_GET_STATUS |

Canonical INFORM core [D]:

NOP
CLEAR I050 20
SUB P070 P070 / SUB P071 P071
SETE P070 (3) 100000                                  ' 100 mm Z offset (micron units)
MOVJ C00000 VJ=50.00
CALL JOB:MM_INIT_SOCKET ARGF"192.168.170.22;50000;1"  ' once only
MOVJ C00001 VJ=50.00 PL=0                             ' image-capture pose
CALL JOB:MM_OPEN_SOCKET
CALL JOB:MM_START_VIS ARGF"1;0;2;30;80"
IFTHENEXP I080<>1102 / PAUSE / ENDIF
CALL JOB:MM_GET_VISDATA ARGF"1;51;52"
IFTHENEXP I052<>1100 / PAUSE / ENDIF                  ' 1003 no cloud in ROI, 1002 no result
CALL JOB:MM_CLOSE_SOCKET
CALL JOB:MM_GET_POSE ARGF"1;71;61;62"
MOVJ C00002 VJ=50.00
SFTON P070 / MOVL P071 V=166.6 PL=0 / SFTOF           ' approach via shift
MOVL P071 V=50.0 PL=0                                 ' pick
END

### 13.7 Cross-brand differences that bite

All [D]:

| Difference | Detail |
| --- | --- |
| Time_Out units | FANUC and YASKAWA = minutes; ABB and KUKA = seconds |
| Explicit socket open/close | ABB and YASKAWA yes; FANUC and KUKA no |
| Protocol format | FANUC V7–V9 HEX big-endian; FANUC V10 HEX little-endian; ABB/KUKA HEX little-endian; YASKAWA ASCII |
| Port range | FANUC V10: 1–32767; others recommend 50000+ |
| Robot-side error strings | FANUC: MM:Robot_Internal_Error [X], MM:Robot_Socket_Closed, MM:Robot_Argument_Error [X], MM:Robot_CMD_Error. YASKAWA: FATAL:MM:INTERNAL_ERROR:…, SOCKET_OPEN_ERROR, SOCKET_CONNECT_ERROR, SOCKET_SEND_ERROR, SOCKET_SELECT_ERROR, SOCKET_RECV_ERROR, SOCKET_RECV_TIMEOUT, SOCKET_CLOSED, ARGUMENT_MISSING, ARGUMENT_INVALID |

MM:Robot_CMD_Error means the command code was not found, or the code received by the background program does not match what the vision system sent — i.e. a calling-sequence violation (§12.5) [D].

## 14. Error handling and recovery

### 14.1 Design the failure modes first

A machine-tending cell that stops on every anomaly has poor OEE; one that retries blindly breaks tools. Classify every failure into one of four responses [E]:

| Class | Response | Examples |
| --- | --- | --- |
| Retry — transient, no state change | Re-trigger vision, up to N times, then escalate | Capture timeout, 1019 execution timeout, 1047 capture-completion timeout |
| Skip — this part is unusable, others may not be | Discard this pose, take the next; if none left, treat as class Wait | 1030 unreachable waypoint, 1036 collision detected, 1035 invalid pick point |
| Wait — nothing to do, but nothing is broken | Signal the operator/PLC, hold, poll | 1002 no vision result, 1003 no point cloud in ROI, 1044/2039 no vision point |
| Stop — integrity is in doubt | Controlled stop, alarm, require human intervention | 1023 failed to connect to camera, 3099 socket failure, 1015 project execution error, any unexpected code |

Write this table into the robot program as an explicit dispatch, not as a chain of IF status <> 1100 THEN PAUSE. FANUC's own MM_S6_Viz_ErrorHandle example is the pattern to follow [D].

### 14.2 Status code dispatch (machine-tending subset)

| Code | Meaning [D] | Class [E] | Robot action [E] |
| --- | --- | --- | --- |
| 1100 | Get vision result successfully | — | Proceed |
| 1102 | Project triggered successfully | — | Proceed to fetch |
| 1103 | Obtained planned path successfully | — | Execute waypoints |
| 1106/1107/1108 | DO list / recipe / dimensions OK | — | Proceed |
| 1002 | No vision result | Wait | "Infeed empty" to PLC; hold; poll |
| 1003 | No point cloud in ROI | Wait | Same; also check for a displaced bin |
| 1007 | Project is running | Retry | Wait 200 ms, retry once; if persistent → Stop (double-trigger bug) |
| 1011 | Project ID does not exist | Stop | Configuration error |
| 1012/1013/1014 | Recipe ID missing / not configured / switch failed | Stop | Changeover configuration error (§16) |
| 1015 | Project execution error | Stop | Look in the Vision log Console tab for the CV-Exxxx detail |
| 1016 | Data does not match the Output step's Port Type | Stop | You called the wrong command for the Output type (§11.1) |
| 1017 | Failed to map label string to numbers | Stop | Labels must be integer-formatted [D] |
| 1019 | Execution timeout | Retry | Retry once, then Stop |
| 1020 | Project not started | Stop | Calling-sequence violation — 102 before 101 |
| 1023 | Failed to connect to camera | Stop | Camera / network / power fault |
| 1026 | Invalid pose type | Stop | Bad robot pose type argument |
| 1027 | Runtime error at Path Planning step | Stop | Planning configuration fault |
| 1030 | Robot cannot reach the waypoint | Skip | Next pose; count consecutive failures |
| 1033 | Motion singularity error | Skip | Next pose |
| 1035 | Invalid pick point | Skip | Next pose |
| 1036 | Robot collision detected | Skip | Next pose; if all fail, escalate — likely a mis-modelled scene |
| 1044 | No vision point for the Vision Move step | Wait | As 1002 |
| 1046 | Invalid tool | Stop | Tool config mismatch |
| 1047 | Timeout waiting for capture completion | Retry | Retry; if repeated, check the camera link speed |
| 2001–2047 | Mech-Viz equivalents (Appendix A) | as above | Mirror the Mech-Vision mapping |
| 3099 | Failed to open socket (used in every official example program, but absent from the official code table) [?] | Stop | Network / interface service not running |
| 3005 | Communication Component: Mech-Vision timed out | Retry→Stop |  |
| 3007 | Data acknowledge from client timed out | Stop | Fieldbus handshake fault (§12.5) |
| 4002 | Robot: Euler angle type not supported | Stop | Wrong brand/convention selected |
| 9100 | System status obtained | — | Check the three sub-status fields |

Full tables in Appendix A.

### 14.3 Distinguishing "empty" from "broken"

The single most valuable thing the robot program can do [E]:

CASE status OF
    1100:              -> normal
    1002, 1003, 1044:  -> DO[infeed_empty] = ON ; message "Load parts" ; wait ; retry
    1030,1033,1035,1036: -> pose_retry = pose_retry + 1
                          IF pose_retry < 3 THEN retry vision ELSE escalate
    ELSE:              -> DO[vision_fault] = ON ; alarm with the code number
ENDCASE

Show the numeric status code on the pendant or HMI. A maintenance technician who can read "1003" off the screen finds the displaced bin in a minute; "Vision Error" costs an hour.

### 14.4 Retry policy that does not break tools

[E]:

- Never retry a motion into the machine. Retry only the vision portion.

- Cap consecutive vision retries (3 is typical) and cap consecutive skip events (e.g. 5 poses) before escalating; a cell that skips 40 poses is telling you the scene model is wrong.

- After any Skip-class failure, re-capture rather than reusing the old result — the scene may have shifted when the previous attempt disturbed the pile.

- On any Stop-class failure, retract to a known safe position outside the machine envelope before alarming, if the robot is not already there and it is safe to move.

- Log every failure with its code, timestamp and cycle counter. Vision faults cluster; the pattern is the diagnosis.

### 14.5 Where to look when it goes wrong

| Symptom | First look |
| --- | --- |
| Status code returned | Appendix A, then the Vision log Console tab ("the final TCP and status codes output by the vision system") [D] |
| 1015 with no detail | Vision log Console tab for CV-Exxxx; the interface only reports the generic 1015 [D]. (CV-E0000 specifically is not documented — do not guess a meaning [?]) |
| Robot alarms MM:Robot_CMD_Error | Calling sequence (§12.5) [D] |
| Yellow step warnings | Look upstream for an empty list (§8.16) |
| Poses look 180° out | The documented 180°-about-X rotation and quaternion→Euler conversion applied when Output-step poses become robot TCP for commands 100/102 (§11.4) [D]; then Flip Poses' Axes configuration (§8.11) |
| Picks are consistently offset | Calibration residual (§6.5), then TCP, then pick point definition |
| Works cold, drifts warm | Thermal drift — deploy drift auto-correction [D] |
| Intermittent, worse when the tray is nearly empty | Depth range or 3D ROI clipping the bottom layer (§5.4) |

## 15. Cycle time engineering

### 15.1 The published decomposition

Mech-Mind's cycle-time guide decomposes the cycle into image capturing → vision processing → path planning & robot motion, and gives two hard numbers [D]:

- The Capture Images from Camera step "usually takes about 40 % of the time" — that is 40 % of the vision project's runtime, not of the whole cell cycle. (The overall-approach page publishes no percentage split for the full cycle.)

- Wait time for vision results should be < 50 ms "to ensure that the system can respond quickly."

- Normal camera data transmission: 700–800 Mbps — verify the link is truly gigabit.

### 15.2 Levers, in the order that pays

All [D] unless noted:

Capture side (the largest single share of vision-project time)

- Minimise exposure time; use Reflective fringe coding rather than stacking 2–3 exposures where the camera supports it.

- Update camera firmware.

- Trim 3D ROI and depth range.

- Ensure a gigabit link (IPC LED orange/green, §3.3).

- Last resort: change camera.

Vision processing

- Do not wire the 2D colour port if it is unused.

- Use 3D ROI extraction to drop excess cloud; downsample where density is not critical.

- Coarse-to-fine 3D matching.

- Disable debug output in production — and turn off Send Point Cloud to External Service unless collision detection needs it (§11.3).

- Non-deep-learning steps scale with CPU multi-core, not GPU — spec the IPC accordingly.

Planning and motion

- Enable "Capture-and-Move" — "robot to move immediately after camera exposure" — so processing overlaps motion. This is the biggest structural win in an eye-to-hand cell.

- Reuse one vision result for multiple picks where the scene does not change between them (a kitted tray is the ideal case).

- Simplify gripper and scene models to cut collision computation.

- Optimise intermediate waypoints and reduce 6th-axis rotation.

- Increase blend radius on non-critical moves.

Protocol level [E]

- Use robot pose type = 2 with 201, or type 3 with 101, so planning starts from a taught capture pose and can run before the robot arrives — the docs explicitly note this "allows planning ahead and shortens cycle time" [D].

- Set 101's expected count to 0 and fetch once, rather than looping 105 per waypoint [D].

- Trim the result list (§7.9): fewer vision points means less planning work.

### 15.3 A realistic budget

For a NANO ULTRA single-pick machine-tending cell [E], with the documented capture time of 0.5–0.9 s [D]:

| Segment | Typical | Notes |
| --- | --- | --- |
| Capture | 0.5–0.9 s | Reflective mode single pass; 2× exposure roughly doubles it |
| Vision processing | 0.3–1.5 s | The reference project runs 37 steps in 1.50 s total |
| Path planning | 0.1–0.5 s | Grows with vision point count and model complexity |
| Robot motion (bin → machine → bin) | 4–15 s | Dominated by distance and machine door geometry |
| Machine handshake waits | 0.5–3 s | Chuck, door, seat check |

Vision is rarely the bottleneck once the capture is tuned — but it is always the thing blamed, because it is the newest component. Measure and publish the split at commissioning so later arguments are about data [E].

## 16. Changeover and multiple part numbers

### 16.1 Parameter recipes — the documented mechanism

Purpose: "switch between parameter recipes in one project to make it applicable to various scenarios" [D].

Create [D]: Project Assistant tab → Parameter Recipe panel → settings icon → Add Recipe, name it. Then select a Step in the left panel and choose that step's parameters on the right to enrol them in the recipe.

Switch [D]:

- Manually — the dropdown in the Parameter Recipe panel; "will take effect when you run the project."

- Automatically — with Standard Interface or Adapter, "you should run a recipe-switching command on the robot side, and Mech-Vision will switch the parameter recipe automatically according to the command it received". Use command 103 (or pass the recipe ID as an argument to 201). You must specify the recipe ID, not the name; IDs are the leftmost column of the Parameter Recipe Editor table, positive integers 1–99.

Which parameter types are eligible, and whether there is a cap on recipe count, are not documented [?].

### 16.2 Choosing a changeover strategy

| Strategy | When | Mechanism |
| --- | --- | --- |
| One project, parameter recipes | Same part family, differing dimensions/tolerances; same fixture | 103 before 101 [D] |
| One project, multiple target objects | Genuinely different geometries in the same tray | Recognise all, use labels to route; the Select Target Object dropdown selects one, so several projects or several recognition steps may be needed [E] |
| Multiple projects in one solution | Different cameras, ROIs or capture parameters per part | 101 with a different project ID |
| Multiple solutions | Different cells or radically different applications | 104 Switch Mech-Vision Solution [D] |
| Operator-driven | Manual changeover, no PLC integration | Production Interface target object management (§17.1) [D] |

### 16.3 Changeover checklist

For each new part number, record and verify [E]:

- ☐ Target object: point cloud model, symmetry, pick points, collision model.

- ☐ Camera configuration parameter group (finish may differ).

- ☐ 3D ROI and depth range (part height differs → depth range differs).

- ☐ Angle gate threshold (a short stubby part tolerates more tilt than a long shaft).

- ☐ Sort rule and row interval (tray pitch differs).

- ☐ Trim Input List output size.

- ☐ Path planning scene objects (different fixture or nest).

- ☐ Gripper recipe / DO mapping on the robot side, keyed to the label.

- ☐ Recipe ID assigned and documented — the robot program sends the ID.

- ☐ Fixture/nest teach positions on the robot.

- ☐ First-off validation: 10 consecutive successful load cycles.

Keep a changeover matrix in the project folder: part number → recipe ID → target object → gripper program → nest teach position. This document is what makes the cell operable by someone other than its author [E].

## 17. Deployment and production

### 17.1 Production Interface

The operator-facing runtime shell, without the full development IDE. Documented capabilities [D]:

- Real-time production status, production results, project execution status.

- Production logs and alert records.

- Vision system accuracy drift monitoring.

- Target object management — switch between target object types during production, add new target objects, show target object detail. This is the operator-level part-number changeover surface for machine tending.

- User authentication (login/logout, account switching, password reset) and solution access control.

- Backup file management, in-interface troubleshooting, operation-guide access.

Set it up with the Production Interface Configurator; entry is via login [D]. Default accounts, roles and passwords are not documented on the reachable pages [?] — establish and record your own.

Practice [E]: configure the Production Interface before handover and train the operator on exactly three actions: read the status, switch the part number, and export a log for support. Do not leave the development IDE as the production UI; an operator with the full step graph will eventually change a parameter.

### 17.2 Autoload and start-up

- Autoload Solution exists as a documented feature (alongside Create/Save Solution, solution access control, switching rules, and project autoload) [D]; the exact semantics — auto-open only, or auto-open plus auto-run — are unconfirmed [?].

- There is no documented mechanism for launching Mech-Vision / Mech-Viz automatically on Windows boot [?]. Autoload governs what loads once the application starts, not application launch. If a cell must survive an unattended power cycle, arrange application launch yourself (a Windows startup shortcut or scheduled task) and prove it by pulling the power at commissioning [E].

- Enable Auto enable interface service when opening the solution in Robot Communication Configuration so the TCP server is live without an operator clicking the toggle [D].

- Have the robot poll 901 before the first cycle after any restart and refuse to run unless Mech-Vision, Mech-Viz (if used) and the camera all report ready [E].

### 17.3 Licensing

- Mech-Mind offers both a hardware dongle and a software licensing code [D].

- Plug full license dongles into USB ports before installing software version 1.6.0 or above [D].

- Path Planning and Mech-Viz features require Mech-Viz installed and licensed [D] — a cell that plans in Mech-Vision still needs the Mech-Viz licence.

- Activation procedure, validity checking, and behaviour on dongle removal are not documented on the reachable pages [?]. Practice [E]: fit an internal USB port or a locking cover so the dongle cannot be borrowed, and record the licence details in the handover pack.

### 17.4 Backups

Only the Production Interface's backup file management is documented [D]; there is no separate backup procedure page [?]. Treat this as practice [E]:

| Back up | When | Where |
| --- | --- | --- |
| Whole solution folder (Mech-Vision projects, Mech-Viz project, robot config, Calibration folder) | Before every change; after every accepted change | Versioned, off-machine, with a dated folder name |
| Camera configuration parameter groups | With the solution |  |
| Target object definitions | With the solution |  |
| Robot program (full controller backup / image) | Before and after every change | Brand-standard backup |
| property_config (if using 207/208) | With the solution |  |
| Calibration result + error report screenshot | After each calibration | Keeps a record of drift over the cell's life |
| Known-good reference: step count, execution time, calibration error | At acceptance | The regression baseline |

The documented project file structure includes vision_project.vis, a Calibration folder, and config files such as depth_background.png, depth_roi.json, roiBoundary.json [D] — copy the folder, not selected files.

### 17.5 Handover pack

[E] — a machine-tending cell should be handed over with:

- This handbook, with the cell-specific appendix filled in (Appendix E).

- Network diagram: IPs, ports, subnets, protocol format, NIC assignments.

- Interface document: which commands the robot sends, in what order, with what arguments, and the status-code dispatch table as implemented.

- Changeover matrix (§16.3).

- Calibration record with the error figure and date.

- Cycle-time split as measured (§15.3).

- Backup media plus restore instructions.

- Spares list: camera, dongle, cables, calibration board.

- Operator one-pager: start, stop, changeover, what the status codes on the HMI mean, when to call for help.

- Support-ticket template: software versions, camera model/ID/firmware, IPC spec, language settings, log export, and a project that reproduces the issue — the documented list of what Mech-Mind support needs [D].

### 17.6 Upgrades — read before you touch a running cell

Documented cautions [D]:

- 1.7.0: project migration is mandatory — projects built with earlier versions must be migrated after upgrading.

- 1.7.2: legacy deep-learning steps are automatically replaced with Deep Learning Model Package Inference.

- Version lock: Mech-Center 1.7.0 must be used with Mech-Vision and Mech-Viz 1.7.0 or above.

- Mech-Viz 1.7.0 renamed robot models → you must re-select the robot from the robot model library after upgrading.

- Target objects configured in 2.1.2 cannot be used with software earlier than 2.1.2.

- Step categories, the Filter → Filter Data by Boolean Value rename, the polarity parameter rename, and Port Type → Output Type all changed between 2.1.x and 2.2.x.

Backup requirements, downgrade support, and the Mech-Eye SDK ↔ Suite compatibility matrix are not stated [?].

Practice [E]: never upgrade a producing cell without (a) a full backup, (b) a scheduled window, (c) a re-run of the calibration accuracy check, (d) a re-verification of collision models and robot model selection, and (e) a 50-cycle soak before releasing to production. On a machine-tending cell an invalidated robot model silently changes collision behaviour — the failure mode is a crash, not an error message.

## 18. Commissioning and acceptance

### 18.1 Vision acceptance

| # | Test | Pass criterion |
| --- | --- | --- |
| V1 | Camera link speed | 1 Gbps or better (IPC LED orange/green) [D] |
| V2 | Point cloud completeness, worst-case part | Complete cloud on graspable surface and matching features |
| V3 | Point cloud stability | ≤ 3 mm fluctuation over 10 static captures [D] |
| V4 | Calibration error | < 2 mm at 100 % for machine tending [D] |
| V5 | Euler-angle consistency along the calibration path | < 1° [D] |
| V6 | Recognition rate, full tray | ≥ 99 % over 100 captures [E] |
| V7 | Recognition rate, last layer / near-empty | ≥ 99 % over 50 captures [E] |
| V8 | False positive rate | 0 accepted poses on an empty tray [E] |
| V9 | Empty-tray behaviour | Returns 1002/1003, not a fault [D] |
| V10 | Execution time | Within the takt budget; record the number |
| V11 | Metadata alignment | Label and dimensions correct for each pose across 20 sorted results [E] |
| V12 | Sort order | Matches the documented tray sequence, operator-legible |

### 18.2 Motion acceptance

| # | Test | Pass criterion |
| --- | --- | --- |
| M1 | Simulated cycle, full bin | Collision-free at production speed |
| M2 | Simulated cycle, near-empty bin | Collision-free |
| M3 | Collision model fidelity | Gripper, held part, machine door, chuck, fence all modelled |
| M4 | Reach at the extremes of the bin | No singularity, no axis limit |
| M5 | Held-part collision check | Enabled (§8.13) and demonstrated with a long part |
| M6 | Dry cycle, no part | Full cycle including handshake, no collision |
| M7 | Single-part cycle | Loads, seats, clamps, machine starts |

### 18.3 Integration acceptance

| # | Test | Pass criterion |
| --- | --- | --- |
| I1 | 901 at start-up | All three sub-statuses report ready |
| I2 | Calling sequence | No MM:Robot_CMD_Error over 100 cycles |
| I3 | Status dispatch | Each class in §14.1 demonstrated by injecting the condition |
| I4 | Empty infeed | Cell holds, signals, resumes when refilled — no alarm |
| I5 | Unreachable pose | Skips to the next pose; escalates after the cap |
| I6 | Camera unplugged mid-cycle | Controlled stop, alarm with code, robot safe outside the machine |
| I7 | IPC power cycle | Cell recovers to a runnable state without a laptop (§17.2) |
| I8 | Wrong part in the tray | Detected by label, rejected, no machine damage |
| I9 | Changeover | Recipe switch by ID, first-off correct, 10 good cycles |
| I10 | Soak | 500 cycles (or one shift) with recorded pick success and cycle time |

### 18.4 Acceptance metrics to publish

[E], since Mech-Mind publishes only a ">99 % picking success rate" claim for machine tending and no cycle-time or accuracy figures [D]:

- Pick success rate over the soak (target ≥ 99 %).

- Mean and worst-case cycle time, with the vision/motion split.

- Recognition rate and false-positive rate.

- Calibration error at acceptance, with the date.

- Number of operator interventions per shift, by cause.

## 19. Maintenance and troubleshooting

### 19.1 Routine schedule

[E], informed by the documented environmental constraints [D]:

| Interval | Task |
| --- | --- |
| Every shift | Clean the camera window; confirm the camera bracket witness mark; check the infeed for chips; read the Production Interface drift indicator |
| Weekly | Review the fault log by status code; check pick success trend; verify the IPC has free disk space |
| Monthly | Calibration accuracy spot check (a taught reference part / calibration sphere); export and archive logs; verify backups restore |
| Quarterly | Recheck TCP; inspect gripper wear (compliance change moves the effective pick point); review the changeover matrix against reality |
| Annually | Full recalibration; controller and IPC backup refresh; review software version against Mech-Mind release notes |
| After any impact, robot repair, robot zeroing, camera swap, or tooling change | Redo hand-eye calibration and re-run §18.1 V4–V5 [E] |

### 19.2 Log locations

[D]:

| Component | Path |
| --- | --- |
| Mech-Vision | \logs under the Mech-Vision install dir (or Open folder in the Log panel) |
| Mech-Viz | \logs under the Mech-Viz install dir |
| Mech-Center (legacy) | \logs under the Mech-Center install dir |
| Deep learning (.dlkpack / .dlkpackC) | \dl_sdk_log under the Mech-Vision install dir |
| Deep learning via Mech-Center | \dl_sdk_log under the Mech-Center install dir |
| Deep learning (legacy .pth / .dlkmp) | \resource\deeplearning_server\logs under Mech-Vision |
| Crash dumps | .dmp files in the Mech-Vision / Mech-Viz / Mech-Center install dirs |
| Mech-Eye Viewer software log | Mech-Eye SDK install path (Open log folder) |
| Camera device log | Mech-Eye Viewer → Device log tab (plain or encrypted) |

Camera log detail [D]: levels [i] / [W] / [C] / [F]; filenames encode timestamps (00105_20221117171503_887.log); a trailing .1 marks the earlier file; camera storage is limited and old logs are deleted, so export promptly. Sync the camera time via Camera Controller on first use.

Mech-Vision log retention defaults to 7 days, configurable up to 99 days at Settings → Options → Common → Log settings [D]. On a production cell, raise it — a fault that recurs every three weeks is invisible with a 7-day window [E].

### 19.3 Troubleshooting playbook

| Symptom | Likely cause | Check, in order |
| --- | --- | --- |
| Recognition rate falls over weeks | Lens film; drift; lighting change | Clean lens → drift indicator → re-run V2/V3 → calibration check |
| Recognition fails only on the bottom layer | Depth range or 3D ROI clipping | §5.4; re-tune with an empty tray |
| Recognition fails only on the top layer of a full tray | Overexposure at short range; depth range upper limit | §5.2 exposure; depth range |
| Random pose 180° out | Symmetry / axis normalisation | Flip Poses' Axes config (§8.11); target object symmetry (§10.3) |
| Robot picks 40 mm high or low | Pick point definition; TCP; calibration | Pick point in the target object editor → TCP → calibration error |
| Right pose, wrong gripper program | Metadata misalignment | §7.7 — check every Sort/Filter/Trim has matching Reorder/port count |
| Works in simulation, collides in reality | Scene model incomplete; point cloud collision off; opaque pallet | §10.4; enable point cloud collision; use Point cloud column |
| Intermittent 1019 / 1047 | Camera link, exposure stack, IPC load | Link speed → reduce exposures → check IPC CPU load and Windows Update state |
| MM:Robot_CMD_Error | Calling-sequence violation | §12.5; check for a stray second 101 or a 102/105 mix |
| Cell hangs waiting for vision | Interface service off; socket left open; timeout mismatch | Toolbar toggle blue? → socket open/close pattern (ABB/YASKAWA) → Time_Out units per brand (§13.7) |
| Everything worked yesterday, nothing today, after an upgrade | Robot model renamed; step renames; target object version | §17.6 |
| Nuisance faults after a Windows restart | Windows Update re-enabled; firewall re-enabled; app not launched | §3.3, §17.2 |

### 19.4 Getting help

Export the log and open a ticket with: software versions, camera model / ID / firmware, IPC spec, language settings, the exported log, and a project that reproduces the issue [D]. For a 1015, include the CV-Exxxx line from the Vision log Console tab — the interface itself cannot tell you more [D].

# Appendices

## Appendix A — Status codes

Ranges [D]:

| Range | Category |
| --- | --- |
| 1001–1099 | Mech-Vision error codes |
| 1100–1199 | Mech-Vision normal status codes |
| 2001–2099 | Mech-Viz error codes |
| 2100–2199 | Mech-Viz normal status codes |
| 3001–3099 | Communication Component error codes |
| 3100–3199 | Communication Component normal status codes |
| 4001–4099 | Robot error codes |
| 4100–4199 | Robot normal status codes |
| 6001–6199 | Status codes that can be customised in Mech-Vision |
| 7001–7099 | Calibration error codes |
| 7100–7199 | Calibration normal status codes |

There is no 5xxx range in the Standard Interface code space [D]. In 1.6/1.7 the 3xxx block was labelled Mech-Center; in 2.x it is Communication Component [D].

### A.1 Mech-Vision errors (1001–1099) [D]

| Code | Description |
| --- | --- |
| 1001 | Unregistered project existed in the solution |
| 1002 | No vision result |
| 1003 | No point cloud in ROI |
| 1005 | Invalid command parameter to start Mech-Vision project |
| 1006 | Invalid pose data |
| 1007 | Project is running |
| 1008 | Digital output signal list not provided |
| 1010 | Number of poses and number of labels do not match |
| 1011 | Project ID does not exist |
| 1012 | Parameter recipe ID does not exist |
| 1013 | Parameter recipe not configured |
| 1014 | Failed to switch parameter recipe |
| 1015 | Project execution error |
| 1016 | Data obtained by the command does not match the Port Type value of the Output Step |
| 1017 | Failed to map the string of label to numbers |
| 1018 | Invalid pose number input |
| 1019 | Execution timeout |
| 1020 | Project not started |
| 1021 | Failed to set object dimensions |
| 1022 | Invalid object dimension settings |
| 1023 | Failed to connect to camera |
| 1024 | Number of poses and list size of custom port data do not match |
| 1025 | Visual image data in use; running cannot proceed |
| 1026 | Invalid pose type |
| 1027 | Runtime error at Path Planning Step |
| 1030 | Robot cannot reach the waypoint |
| 1033 | Motion singularity error |
| 1035 | Invalid pick point |
| 1036 | Robot collision detected |
| 1037 | No available placing position for palletizing |
| 1044 | No vision point for the Vision Move Step |
| 1046 | Invalid tool |
| 1047 | Timeout occurs when waiting for the capture completion |
| 1048 | Box mask recognition error |
| 1049 | Check box dimensions error |
| 1051 | Failed to set pose |
| 1052 | Failed to declare/get global variable |
| 1053 | Failed to execute the switch solution instruction |

### A.2 Mech-Vision normal (1100–1199) [D]

| Code | Description |
| --- | --- |
| 1100 | Get vision result successfully |
| 1101 | Ready to run |
| 1102 | Project got triggered successfully |
| 1103 | Obtained planned path successfully |
| 1106 | Obtained DO signal list successfully |
| 1107 | Parameter recipe switched successfully |
| 1108 | Object dimensions input successfully |
| 1110 | Pose set successfully |
| 1104 / 1111 | Solution switched / global variable set-or-read — used on the command pages but absent from the official status table [?] |

### A.3 Mech-Viz errors (2001–2099) [D]

| Code | Description |
| --- | --- |
| 2001 | Software not registered |
| 2002 | Project is running |
| 2004 | Robot cannot reach the waypoint |
| 2006 | Invalid command parameter to start Mech-Viz |
| 2008 | Runtime error |
| 2011 | Digital output signal list not provided |
| 2012 | Invalid pose type |
| 2013 | Invalid pose data |
| 2014 | No project set to autoload |
| 2015 | Project not opened |
| 2016 | Failed to set Step parameter value |
| 2017 | Failed to stop execution |
| 2018 | Invalid branch exit port number |
| 2019 | Failed to set branch |
| 2020 | Motion singularity error |
| 2022 | Project not executed, or no results after execution |
| 2024 | Invalid Branch Step ID |
| 2025 | Execution timeout |
| 2026 | Invalid Step ID of index-type Steps |
| 2027 | Invalid Current Index value |
| 2028 | Failed to set index |
| 2030 | Invalid pick point |
| 2031 | Robot collision detected |
| 2032 | No available placing position for palletizing |
| 2036 | Visual Recognition Step not called |
| 2037 | No vision result received from vision service |
| 2038 | No point cloud in ROI |
| 2039 | No vision point for the Vision Move Step |
| 2041 | Failed to get Step parameter |
| 2042 | Failed to get planning result in Vision Move |
| 2043 | Failed to get custom data |
| 2044 | Vision service not registered |
| 2045 | Invalid tool |
| 2047 | Check box dimensions error |

### A.4 Mech-Viz normal (2100–2199) [D]

| Code | Description |
| --- | --- |
| 2100 | Execution completed successfully |
| 2102 | DO signal list obtained successfully |
| 2103 | Started successfully |
| 2104 | Stopped successfully |
| 2105 | Set branch successfully |
| 2106 | Set index successfully |
| 2107 | Set move point for External Move Step successfully |
| 2108 | Set Step parameter value successfully |
| 2109 | Get Step parameter value successfully |

### A.5 Communication Component, robot, calibration [D]

| Code | Description |
| --- | --- |
| 3001 | Invalid command |
| 3002 | Invalid data length or format for command parameter |
| 3005 | Mech-Vision timed out (gRPC service call) |
| 3006 | Unknown error |
| 3007 | Data acknowledge signal from client timed out |
| 3008 | Configuration ID does not exist |
| 3103 | Modbus TCP data cleared successfully |
| 3099 | Failed to open socket — used in every official example program but absent from the official table [?] |
| 4002 | Robot: Euler angle type not supported |
| 7001 | Calibration: parameter error |
| 7002 | Calibration: no calibration flange pose provided |
| 7003 | Calibration: calibration joint positions not provided |
| 7004 | Calibration: robot failed to reach the calibration point |
| 7100 | Calibration: robot moved to the calibration point successfully |
| 7101 | Calibration: pose received from Mech-Vision successfully |

3007 fires when, on PROFINET / EtherNet/IP, the client fails within the timeout (default 10 s) to reset Data_Acknowledge to 0 before new pose data is sent, set it to 1 after, clear NOTIFY via CLEAR_NOTIFY before the next notify message, or reset EXPOSURE_COMPLETE via RESET_EXPOSURE before the next exposure [D].

Legacy 1.6 codes dropped in 2.x but still seen in field notes: 3003 client disconnected, 3004 server disconnected, 3100 client connection normal, 3101 server connection normal, 3102 waiting for client to connect [D].

## Appendix B — Command quick reference and legacy names

| # | 2.x name | 1.7.x name |
| --- | --- | --- |
| 100 | Trigger Mech-Vision Project and Get Results | (2.x addition) |
| 101 | Trigger Mech-Vision Project | Start Mech-Vision Project |
| 102 | Get Vision Results | Get Vision Target(s) |
| 103 | Switch Mech-Vision Parameter Recipe | Switch Mech-Vision Recipe |
| 104 | Switch Mech-Vision Solution | (2.x addition) |
| 105 | Get Planned Path from Mech-Vision | Get Result of Step "Path Planning" |
| 106 | Get Gripper DO List from Mech-Vision | (2.x addition) |
| 110 | Get Custom Data from Mech-Vision | Get Custom Output Data from Mech-Vision |
| 111 | Get Vision Move Data from Mech-Vision | (2.x addition) |
| 200 | Trigger Mech-Viz Project and Get Planned Path | (2.x addition) |
| 201 | Trigger Mech-Viz Project | Start Mech-Viz Project |
| 202 | Stop Mech-Viz Project | (same) |
| 203 | Set Exit Port for Branch by Msg | Select Mech-Viz Branch |
| 204 | Set Current Index | Set Move Index |
| 205 | Get Planned Path from Mech-Viz | Get Planned Path |
| 206 | Get Gripper DO List from Mech-Viz | Get DO List |
| 207 / 208 | Read / Set Mech-Viz Step Parameter | (same) |
| 210 | Get Vision Move Data or Custom Data from Mech-Viz | Get Waypoint with Depalletizing Planning Data |
| 501 | Input Object Dimensions | (same) |
| 502 | — | Input TCP to Mech-Viz (External Move); absent from the 2.2.1 list, though status 2107 survives [?] |
| 503–507 | Input Poses / global variables | (2.x additions) |
| 601 | Get Message from Notify Step | Notify |
| 701 | Calibration | (same) |
| 901 | Get System Status (3 status fields) | Get Software Status (1101 = ready) |

All [D].

## Appendix C — Robot routine cross-reference

| Function | FANUC | ABB | KUKA | YASKAWA |
| --- | --- | --- | --- | --- |
| Init comms | MM_INIT_SKT | MM_Init_Socket | MM_Init_Socket | MM_INIT_SOCKET |
| Open / close socket | — | MM_Open_Socket / MM_Close_Socket | — | MM_OPEN_SOCKET / MM_CLOSE_SOCKET |
| Trigger Mech-Vision | MM_START_VIS | MM_Start_Vis | MM_Start_Vis | MM_START_VIS |
| Get vision result | MM_GET_VIS | MM_Get_VisData | MM_Get_VisData | MM_GET_VISDATA |
| Store pose (TCP) | MM_GET_POS | MM_Get_Pose | MM_Get_Pose | MM_GET_POSE |
| Store joint positions | MM_GET_JPS | MM_Get_Jps | MM_Get_Jps | MM_GET_JPS |
| Switch recipe | MM_SET_MOD | MM_Switch_Model | MM_Switch_Model | MM_SET_MODEL |
| Get planned path (Vision) | MM_GET_VISP | MM_Get_VisPath | MM_Get_Vispath | MM_GET_VISPATH |
| Custom data | MM_GET_DY_DT / MM_GET_DYPOS | MM_Get_DyData / MM_Get_DyPose | MM_Get_Dy_Data / MM_Get_DyPose | MM_GET_DYDATA / MM_GET_DYPOSE |
| DO list | MM_GET_DL / MM_SET_DL | MM_Get_DoList / MM_Set_DoList | MM_Get_DoList / MM_Set_DoList | MM_GET_DOLIST / MM_SET_DOLIST |
| Trigger / stop Mech-Viz | MM_START_VIZ / MM_Stop_Viz | MM_Start_Viz / MM_Stop_Viz | MM_Start_Viz / MM_Stop_Viz | MM_START_VIZ / MM_Stop_Viz |
| Branch / index | MM_SET_BCH / MM_SET_IDX | MM_Set_Branch / MM_Set_Index | MM_Set_Branch / MM_Set_Index | MM_SET_BRANCH / MM_SET_INDEX |
| Get planned path (Viz) | MM_GET_VIZ | MM_Get_VizData | MM_Get_VizData | MM_GET_VIZDATA |
| Step parameter r/w | MM_GET_PROP / MM_SET_PROP | MM_Get_Property / MM_Set_Property | MM_Get_Property / MM_Set_Property | MM_GET_PROPERTY / MM_SET_PROPERTY |
| Object dimensions | MM_SET_BS | MM_Set_BoxSize | MM_Set_BoxSize | MM_SET_BOXSIZE |
| Notify | MM_GET_NTFY | MM_Get_Notify | MM_Get_Notify | MM_GET_NOTIFY |
| Calibration | MM_CALIB | MM_Calib | MM_Calib | MM_CALIB |
| System status | MM_GET_STAT | MM_Get_Status | MM_Get_Status | MM_GET_STATUS |
| One-shot trigger+fetch | — | MM_Lite_Vis / MM_Lite_Viz | — | — |
| Pose → base frame | — | — | MM_Get_Wobj | — |
| Comms test program | MM_COMTEST | MM_Com_Test.mod | MM_COMTEST.src | MM_COMTEST |

All [D].

## Appendix D — Naming and project hygiene conventions

[E] — adopt or replace, but write it down and follow it:

| Item | Convention | Example |
| --- | --- | --- |
| Solution | <Cell>_<Machine> | MechMaster |
| Vision project | <Part>_<Operation> | Drill_Tap |
| Target object | <partnumber> | t1 → prefer PN12345_blank |
| Camera parameter group | <part>_<finish> | t1_machined_reflective |
| Calibration group | `_<ETH | EIH>` |
| Step instance | <Role>_<Detail> | Gate_Tilt_PreFlip, Sort_Layer_Z, Trim_OnePerCycle |
| 3D ROI | <Region> | ROI_Infeed, ROI_Nest |
| Recipe | <PartNumber> with a fixed ID | PN12345 = ID 3 |
| Robot program | MT_<Cell>_<Function> | MT_LATHE1_MAIN, MT_LATHE1_VISION |
| Register block | Reserve and document a contiguous block | R[50–59] status, PR[60–69] vision poses |

## Appendix E — Cell-specific record (fill in per installation)

| Item | Value |
| --- | --- |
| Cell / machine |  |
| Archetype (§1.2) |  |
| Robot make / model / controller / software version |  |
| Camera model / SN / firmware |  |
| IPC model / OS / software versions (Vision, Viz, Eye SDK) |  |
| Mounting mode (ETH/EIH) and working distance |  |
| Camera IP / IPC IP / robot IP / subnet |  |
| Standard Interface port / protocol format |  |
| Solution name / project name(s) / project IDs |  |
| Recipe table (ID → part number) |  |
| Calibration date / method / error at 100 % |  |
| Tolerance budget (§1.3) |  |
| Measured cycle time and split (§15.3) |  |
| Known-good step count / execution time |  |
| Backup location |  |
| Licence type / dongle serial |  |
| Acceptance date / signed by |  |

## Appendix F — Items to verify on your build

Every claim marked [?] in this handbook, collected:

- CV-E0000 — no documented meaning; CV-Exxxx is a Mech-Vision internal step error surfaced as status 1015 only.

- exit_code — no such field exists anywhere in the Standard Interface; if your project emits one, it is an Adapter-layer or step-level field.

- 3099 (failed to open socket) — used in every official example program, absent from the official status table.

- 1104 and 1111 — used on the command pages, absent from the official status table.

- No 5xxx status range exists.

- Command 502 (Input TCP to Mech-Viz) — present in 1.7.4, absent from the 2.2.1 list, although status 2107 survives.

- UDP Standard Interface command reference — officially "not yet available".

- 3D Target Object Recognition port labels Pick Point IDs / Colored Point Cloud / Merged Point Cloud / Object Point Clouds / Pick Point Labels — not documented under those names; probable mapping in §7.4.

- .tob target object file extension — not documented anywhere reachable.

- "Target Object Confidence" as a parameter name — not documented; see the Target object confidences port and 3D Matching's Result Verification Degree + Confidence Threshold (default 0.3).

- Path Planning output labelled "Error Results" — docs say Filter Results (2.1.2) / Filtered Results (2.2.1).

- The yellow warnings "The nth input is empty. No output. Auto-fill output" / "No output. Auto-fill output" — not documented; nearest documented mechanisms in §8.16.

- A per-run "step count" line format in the Vision log — not documented; only per-step Time: n ms on the block is.

- Parameters for Validate Poses by Included Angles to Reference Direction, Validate Existence of Poses in 3D ROI, Easy Create Poses, Trigger Control by Flag, Reorder by Index List — marked "under maintenance" in 2.1.2; tables here come from 2.2.1.

- Calibration board selection table by camera model / working distance.

- No official "recalibrate when the camera is bumped" statement exists; Quick Camera Replacement prerequisites are undocumented.

- Licence dongle activation, validity check, and removal behaviour.

- No documented Windows autostart for Mech-Vision / Mech-Viz; Autoload Solution semantics unconfirmed.

- Solution backup procedure — only the Production Interface's backup file management is documented.

- Mech-Viz Checker, Counter, Message step parameter references.

- Point-cloud collision default values and the separate volume/point thresholds.

- Mech-Eye per-model parameter defaults (Gap Filling, Fringe Contrast Threshold, Outlier Removal, Surface Smoothing, Distortion Correction, Anti-Blur).

- Upgrade backup requirement, downgrade path, and the Mech-Eye SDK ↔ Suite compatibility matrix.

- Machine-tending cycle times and mm accuracy are not published by Mech-Mind; only a ">99 % picking success rate" claim.

- Production Interface default accounts, roles and passwords.

- A Solution Library template literally named "machine tending" or "CNC loading" is not confirmed; the confirmed equivalents are General Workpiece Recognition and the tutorial's two Vision-Guided Loading … templates.

- NANO ULTRA mounting mode — documented as eye-in-hand in the model comparison while the machine-tending solution page recommends PRO/LSR; confirm ETH support for your SKU. Also confirm which variant you have (350M, 250–500 mm vs 700M, 400–800 mm) — the case label on the reference unit does not say.

- Parameter recipe eligibility rules and any cap on recipe count.

- Which 2.1.x build corresponds to the observed 2.1.1 UI in the reference project — this handbook uses 2.1.2 documentation as the closest published reference.

## Appendix G — Glossary

| Term | Meaning |
| --- | --- |
| Vision point | An object recognised by Mech-Vision: pose, label, dimensions, custom data [D] |
| Waypoint | A point the robot reaches along a planned path: pose, label, motion type [D] |
| Pick point | A grippable position defined in the object reference frame [D] |
| Target object | The configured object definition: point cloud model, symmetry, centre point, pick points, collision model [D] |
| Parameter recipe | A named set of step parameters switchable at runtime by ID [D] |
| Solution | Container for Mech-Vision project(s), one Mech-Viz project, robot config, calibration [D] |
| Standard Interface | Mech-Mind's fixed command protocol where the external device is the client [D] |
| Master-Control | Mode where the vision system commands the robot [D] |
| Adapter | Mode where you define the protocol and formats yourself (Python) [D] |
| ETH / EIH | Eye-to-hand (fixed camera) / eye-in-hand (camera on the flange) [D] |
| Communication Component | The service hosting the Standard Interface server; replaced Mech-Center in 2.x [D] |
| Notify | The step + command 601 mechanism for pushing a message to the robot; 3-second buffer [D] |
| Mapped Indices | The permutation emitted by a sort step, applied via Reorder by Index List [D] |

## Appendix H — Sources

### Standard Interface and robot integration

- TCP/IP Interface Commands (2.2.1) — https://docs.mech-mind.net/en/robot-integration/latest/standard-interface-development-manual/tcp-ip-socket.html

- TCP/IP Interface Commands (1.7.4) — https://docs.mech-mind.net/en/robot-integration/1.7.4/standard-interface-development-manual/tcp-ip-socket.html

- Status Codes and Troubleshooting — https://docs.mech-mind.net/en/robot-integration/latest/standard-interface-development-manual/status-codes-error-troubleshooting.html

- Calling Sequence of Standard Interface Commands — https://docs.mech-mind.net/en/robot-integration/latest/standard-interface-development-manual/control-timing.html

- Standard Interface Development Manual (protocol matrix) — https://docs.mech-mind.net/en/robot-integration/latest/standard-interface-development-manual/standard-interface-development-manual.html

- Communication Modes — https://docs.mech-mind.net/en/robot-integration/latest/communication-basics/communication-modes.html

- Robot Communication Configuration — https://docs.mech-mind.net/en/suite-tutorial/latest/vision-system-communication-configuration.html

- FAQ 13, IP address and port — https://docs.mech-mind.net/en/robot-integration/latest/faq/faq-13.html

- MC / Standard Interface commands (2.1.2) — https://docs.mech-mind.net/en/robot-integration/2.1.2/standard-interface-development-manual/mc-commands.html

- FANUC: commands · setup · examples · MM_S1_Vis_Basic · error messages — https://docs.mech-mind.net/en/robot-integration/latest/standard-interface-robot/fanuc-interface-commands.html

- ABB: commands · setup · MM_S1_Vis_Basic · error messages — https://docs.mech-mind.net/en/robot-integration/latest/standard-interface-robot/abb-interface-commands.html

- KUKA: commands · setup · error messages — https://docs.mech-mind.net/en/robot-integration/latest/standard-interface-robot/kuka-interface-commands.html

- YASKAWA: commands · setup · MM_S1_Vis_Basic · error messages — https://docs.mech-mind.net/en/robot-integration/latest/standard-interface-robot/yaskawa-interface-commands.html

### Mech-Vision steps and tools

- Step Reference Guide (2.1.2) — https://docs.mech-mind.net/en/suite-software-manual/2.1.2/vision-steps/steps.html

- Step Reference Guide (latest) — https://docs.mech-mind.net/en/suite-software-manual/latest/vision-steps/steps.html

- Capture Images from Camera — https://docs.mech-mind.net/en/suite-software-manual/2.1.2/vision-steps/capture-images-from-camera.html

- 3D Target Object Recognition · tool · target object selection · general settings — https://docs.mech-mind.net/en/suite-software-manual/2.1.2/vision-steps/3d-target-object-recognition.html

- 3D Matching basic tuning (confidence) — https://docs.mech-mind.net/en/suite-software-manual/2.1.2/vision-steps/3d-matching-basic-parameters.html

- Target Object Editor · Point Cloud Models and Pick Points — https://docs.mech-mind.net/en/suite-software-manual/2.1.2/vision-tools/target-object-editor.html

- Easy Create Poses — https://docs.mech-mind.net/en/suite-software-manual/2.1.2/vision-steps/easy-create-poses.html

- Validate Poses by Included Angles to Reference Direction — https://docs.mech-mind.net/en/suite-software-manual/2.1.2/vision-steps/validate-poses-by-included-angle-to-reference-direction.html

- Validate Existence of Poses in 3D ROI — https://docs.mech-mind.net/en/suite-software-manual/2.1.2/vision-steps/validate-existence-of-poses-in-3d-roi.html

- Sort 3D Poses V2 — https://docs.mech-mind.net/en/suite-software-manual/2.1.2/vision-steps/sort-3d-poses-v2.html

- Filter / Filter Data by Boolean Value — https://docs.mech-mind.net/en/suite-software-manual/2.1.2/vision-steps/filter.html

- Reorder by Index List — https://docs.mech-mind.net/en/suite-software-manual/2.1.2/vision-steps/reorder-by-index-list.html

- Trim Input List — https://docs.mech-mind.net/en/suite-software-manual/2.1.2/vision-steps/trim-input-list.html

- Trigger Control by Flag — https://docs.mech-mind.net/en/suite-software-manual/2.1.2/vision-steps/trigger-control-by-flag.html

- Flip Poses' Axes — https://docs.mech-mind.net/en/suite-software-manual/2.1.2/vision-steps/flip-poses-axes.html

- Transform Poses — https://docs.mech-mind.net/en/suite-software-manual/2.1.2/vision-steps/transform-poses.html

- Path Planning step · Path Planning Tool · Configure collision detection — https://docs.mech-mind.net/en/suite-software-manual/2.1.2/vision-steps/path-planning.html

- Output (procedure out) — https://docs.mech-mind.net/en/suite-software-manual/2.1.2/vision-steps/procedure-out.html

- Show Point Clouds and Poses — https://docs.mech-mind.net/en/suite-software-manual/2.1.2/vision-steps/show-point-clouds-and-poses.html

- Step Common Parameters · Statuses of Steps · Input/Output of Steps — https://docs.mech-mind.net/en/suite-software-manual/2.1.2/vision-operation-guide/understand-step-common-parameters.html

- Run Project and Debug · Run Steps and View Outputs · Run Project Continuously · Project Toolbar · Log — https://docs.mech-mind.net/en/suite-software-manual/2.1.2/vision-operation-guide/run-project-and-debug.html

- Project file structure — https://docs.mech-mind.net/en/suite-software-manual/2.1.2/vision-operation-guide/project-file-structure.html

- Parameter Recipe Configuration — https://docs.mech-mind.net/en/suite-software-manual/latest/vision-tools/parameter-recipe-configuration.html

- Guide to Production Interface — https://docs.mech-mind.net/en/suite-software-manual/latest/vision-production-interface/production-interface.html

- Auto-Correct Accuracy Drift · Quick Camera Replacement — https://docs.mech-mind.net/en/suite-software-manual/latest/vision-tools/accuracy-error-analysis-tool-drift-correction.html

### Calibration

- Hand-Eye Calibration Guide — https://docs.mech-mind.net/en/suite-software-manual/latest/vision-calibration/calibration.html

- Hand-Eye Calibration tutorial — https://docs.mech-mind.net/en/suite-tutorial/latest/vision-system-calibration.html

- Automatic Calibration, ETH — https://docs.mech-mind.net/1.7/en-GB/SoftwareSuite/MechVision/Calibration/EthAutoCalib.html

- Manual Calibration, ETH TCP touch — https://docs.mech-mind.net/1.7/en-GB/SoftwareSuite/MechVision/Calibration/EthManualCalibTcpTouch.html

- Calibration Board appendix — https://docs.mech-mind.net/1.7/en-GB/SoftwareSuite/Appendix/CalibrationBoard/CalibrationBoard.html

- Calibration Results Check and Analysis — https://docs.mech-mind.net/1.7/ko-KR/SoftwareSuite/MechVision/Calibration/Reference/CalibResultAnalysis.html

- Calibration Troubleshooting and FAQs — https://docs.mech-mind.net/latest/en-GB/SoftwareSuite/MechVision/Calibration/Reference/CalibTroubleshooting.html

### Camera and IPC

- NANO ULTRA-GL specifications — https://docs.mech-mind.net/en/eye-3d-camera/latest/hardware/specifications-nano-ultra.html

- NANO ULTRA specifications (2.3.3) — https://docs.mech-mind.net/en/eye-3d-camera/2.3.3/hardware/specifications-nano-ultra.html

- Camera models and suitable applications — https://docs.mech-mind.net/en/eye-3d-camera/2.6.0/hardware/camera-models.html

- Adjust 3D Exposure Settings — https://docs.mech-mind.net/en/eye-3d-camera/latest/extended-reading/determine-and-adjust-3d-exposure.html

- Advanced Parameters for Depth Map and Point Cloud — https://docs.mech-mind.net/latest/en-GB/MechEye/MechEyeViewer/UsingMechEyeViewer/ParameterAdjustments/DepthMap/DepthMapAdvanced.html

- Adjust Camera Parameters Using Mech-Eye Viewer — https://docs.mech-mind.net/en/eye-3d-camera/latest/genicam/adjust-parameters.html

- IPC Setup — https://docs.mech-mind.net/latest/en-GB/SoftwareSuite/Appendix/Ipc/IpcConfiguration.html

- Project docs: mech_mind_camera_md_docs.md, mech_mind_IPC_STD_DOCS.md (attached to this project)

### Mech-Viz

- Mech-Viz overview · Fundamentals · Project and Solution — https://docs.mech-mind.net/1.7/en-GB/SoftwareSuite/MechViz/BasicConcepts/BasicConcepts.html

- Workflow reference — https://docs.mech-mind.net/1.7/ko-KR/SoftwareSuite/MechViz/Workflow/Workflow.html

- Configure the Workflow (pick-place order) — https://docs.mech-mind.net/1.6/en-GB/SoftwareSuite/MechMindQuickStart/FirstApplication/MechVizSimulatePicking/WorkFlow/WorkFlow.html

- End Effector · Robot Link · Point Cloud collision configuration — https://docs.mech-mind.net/1.6/en-GB/SoftwareSuite/MechViz/Collisions/DetectionSettings/EndEffectorSettings/EndEffectorSettings.html

- Configure Robot Tools and Workobjects — https://docs.mech-mind.net/1.6/en-GB/SoftwareSuite/MechViz/GettingStarted/ToolsAndWorkobjects/ToolsAndWorkobjects.html

### Application guidance

- Robotic Machine Tending with 3D Vision — https://www.mech-mind.com/solution/machine-tending.html

- Vision System Tutorial index — https://docs.mech-mind.net/en/suite-tutorial/latest/index.html

- Vision Solution Design — https://docs.mech-mind.net/en/suite-tutorial/latest/vision-system-design.html

- Workpiece Locating (General Workpiece Recognition) — https://docs.mech-mind.net/en/suite-tutorial/1.8.3/getting-started/first-application-vision.html

- Guide to Improving Application Cycle Time — https://docs.mech-mind.net/en/suite-tutorial/latest/cycle-time/cycle-time-improvement-guide.html

- Error Code Processing — https://docs.mech-mind.net/en/suite-service-manual/latest/troubleshooting/troubshooting-common-issues-error-codes.html

- Collect Information about the Issue (log paths) — https://docs.mech-mind.net/1.7/en-GB/SoftwareSuite/Support/Troubleshooting/MustRead/IssueInfoCollection.html

- Before You Upgrade — https://docs.mech-mind.net/1.7/en-GB/SoftwareSuite/ReleaseNote/General.html

End of handbook. Claims marked [?] in the text are collected in Appendix F — verify them on your build before relying on them in a production cell.